Question 1: Please provide SDG&E’s estimate of annual costs for its proposed Peak Time Rebate (PTR) program.  Your response should include the following cost items,: 1) PTR program administration, 2) customer service, 3) customer education, and 4) billing (calculation of Customer Reference Level or CRL and rebate levels).  Please identify and quantify any other costs, and include all supporting workpapers (electronic and/or hard copy) showing SDG&E’s calculation of the annual PTR costs.

SDG&E Response 1:

The estimates for annual costs for the proposed Peak Time Rebate (PTR) program were provided in the Advanced Metering Infrastructure (AMI) application A.05-03-015.  The attached workpaper was submitted in conjunction with Chapter 5: Testimony of Mark Gaines and details the costs for customer service and outreach for PTR customers. In addition, billing costs were addressed in Chapter 3: Testimony of James Teeter.

A summary of these costs for the years 2008-2010, with cross references to the workpaper, is presented in the following table:

|  |  |  | Costs ($s) |  |
| --- | --- | --- | --- | --- |
| Item | Line #’s | 2008 | 2009 | 2010 |
| 1. Administration | 17 | 0 | 1,556,333 | 1,556,333 |
| 2. Customer Service | 9 | 56,374 | 56,374 | 56,374 |
| 3. Customer Education | 10, 11, 12, 13, 14, 15, 16 | 634,044 | 806,992 | 772,992 |
| 4. Billing * |  | 0 | 0 | 0 |
| Total |  | 690,418 | 2,419,699 | 2,385,699 |

* Per the testimony of James Teeter, there are no incremental annual costs associated with PTR billing, calculation of CRL or rebate levels.  Required upgrades to the computer billing system represent a one-time cost only, with no ongoing annual costs.

Response Prepared By:   Joe Velasquez

Revised Question 2: Referring to Ms. Willoughby's Testimony, Table LW-5, please update the present value of demand benefits ($million of GRC II test year).

SDG&E Response 2:

| Table DRA10 Q2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Present Value of Demand Response Benefits |  |  |  |  |
| (Millions of 2008 $) |  |  |  |  |
| Customer Segment | Capacity | Energy | Total | 2011 MW |
| Residential | $191.1 | $10.9 | $202.0 | 160 |
| Small C&I (<20 kW) | $15.5 | $1.1 | $16.6 | 9 |
| Medium C&I (20- 200 kW) | $78.9 | $2.5 | $81.5 | 69 |
| Large C&I (> 200 kw) | $75.7 | $2.3 | $78.0 | 64 |
| Total | $361.2 | $16.8 | $378.0 | 302 |

Note:  Demand Benefits were updated to reflect 2008 Present Value Dollars

Response Prepared By:  Leslie Willoughby

Question 3: Please provide the demand response (2011 MW, capacity and energy) as shown in Table LW-5 of Ms. Willoughby's Testimony and annual and total present value benefits (millions of 2006$ and 2007$) using the input assumptions/methodologies adopted in Decision (D.) 07-04-043: 1) analysis horizon of 2009 to 2026 (17 years), 2) avoided capacity costs of $52/kW-year, 3) PTR and CPP participation rates as shown in the table below; and SDG&E’s proposed PTR credit and CPP rates in this proceeding.  Please provide all supporting workpapers (electronic and/or hard copy).

For small C&I customers, please separate the demand response in MWs and benefits for PTR and default TOU.

|  | Participation Rates |  |  |
| --- | --- | --- | --- |
| Customer Segment | 2009 | 2010 | 2011 & beyond |
|  | (%) |  |  |
| Residential | 56 | 56 | 56 |
| Small C&I | 100 | 100 | 100 |
| Medium C&I | 69 | 69 | 69 |
| Large C&I | 5 | 5 | 69 |

SDG&E Response 3:

| Table DRA DR10 Q3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Present Value of Demand Response Benefits |  |  |  |  |
| (Millions of 2008 $) |  |  |  |  |
| Customer Segment | Capacity | Energy | Total | 2011 MW |
| Residential | $ 81.5 | $ 5.9 | $ 87.4 | 122 |
| Small C&I (<20 kW) | $ 11.1 | $ 0.8 | $ 11.9 | 9 |
| Medium C&I (20- 200 kW) | $ 43.1 | $ 1.5 | $ 44.6 | 49 |
| Large C&I (> 200 kw) | $ 33.2 | $ 1.1 | $ 34.3 | 46 |
| Total | $ 168.9 | $ 9.3 | $ 178.2 | 227 |

Response Prepared By:  Leslie Willoughby

Question 4: Is SDG&E planning on updating the testimony on demand response benefits in this proceeding to reflect the input assumptions/methodology adopted in D.07-04-043?  If the answer is “yes”, please indicate the date that SDG&E will provide the update.  If the answer is “no”, please explain why SDG&E believes that the Commission should adopt SDG&E’s original input assumptions in its AMI application (A.05-03-015).

SDG&E Response 4:

SDG&E is not planning to update the testimony on demand response benefits in this proceeding to reflect the input assumptions/methodology adopted in D.07-04-043 (AMI decision) because:

The AMI settlement agreement explicitly states that the demand response rates will be litigated in SDG&E's GRC Phase 2, which implicitly means that the demand response impacts will also be litigated because the demand response impacts depend upon the rate design.

Response Prepared By:  Ed Fong

Question 5: Referring to Ms. Willoughby's Testimony, Table LW-2:

  - Please provide a breakdown of the percentages and the number of customers by: 1) the climate zones in SDG&E’s service territory and 2) by customers usage groups.  In addition, please provide the total peak usage (2004, 2005, & 2006) for the “Disadvantage Error > 15%” and “Able to earn Rebate” customer groups.

SDG&E Response 5:

| Percentage of SDG&E Customers by Climate Zone |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year | Coastal | Inland | Mountain | Desert |
| 2004 | 56.9% | 41.8% | 1.1% | 0.2% |
| 2005 | 56.9% | 41.8% | 1.1% | 0.2% |
| 2006 | 56.9% | 41.8% | 1.1% | 0.2% |

Prepared by: Leslie Willoughby

Question 6: Referring to Ms. Willoughby's Testimony, Section III, Customer Reference Level:

- Please explain why SDG&E chose its proposed CRL methods of the “High 3 of 5” for residential customers and “High 3 of 10” for small commercial customers?

- What other methods has SDG&E considered in this proceeding, and if other methods were considered, please explain why they were not adopted?

- Provide a side by side comparison of the information shown in Tables LW-1 to LW-4 from the best three different CRL methods that SDG&E has considered, including the 5 previous like days (adjusted and unadjusted ) as proposed in SDG&E’s AMI application (A.05-03-015).

SDG&E Response 6:

SDG&E analyzed the accuracy of several reference levels using statistics such as total error and bias. The overall goal was a reference level with low total error, low bias, easy for customers to understand, and difficult for a customer to game.  The residential high 3 of 5 day baseline performed well in all categories as did the high 3 of 10 day for the small commercial class.

- SDG&E considered several customer reference levels (CRL). The statistics for the top three CRL that were not selected are presented below in response 6c). The results for the other top 3 CRL are similar to the results for SDG&E’s chosen CRL. The 5-day adjusted per customer CRL did perform slightly better than SDG&E’s recommended CRL. However, the final selection of the CRL took into consideration how easy the CRL was to understand and the ability of SDG&E’s billing system to calculate the CRL. SDG&E also believes that over time when fully understood, that the 5-day adj per customer CRL could be gamed by customers.

- Top 4 Residential Customer Reference Levels considered by SDG&E:

| Residential Reference Levels | Year | Total Error (kWh) | Average Error | Median Error | Total Rebate No DR in Millions $ |
| --- | --- | --- | --- | --- | --- |
| 5-day adj per customer | 2004 | 18,535,757 | 2.0% | 0.1% | -8.4 |
| high 3 of 5 | 2004 | 21,934,922 | 2.4% | -1.5% | -10.0 |
| high 3 of 6 | 2004 | 22,719,762 | -0.2% | -2.6% | -11.4 |
| 5-day adj per event (AMI illustrative) | 2004 | 22,936,519 | 2.3% | -4.0% | -10.5 |
| 5-day adj per customer | 2005 | 17,940,936 | 3.4% | 0.3% | -7.5 |
| high 3 of 5 | 2005 | 26,331,258 | 5.8% | -0.9% | -10.7 |

SDG&E Response 6-Continued:

| 5-day adj per event (AMI illustrative ) | 2005 | 26,970,209 | 3.6% | -3.9% | -12.0 |
| --- | --- | --- | --- | --- | --- |
| high 3 of 6 | 2005 | 27,033,373 | 2.7% | -1.7% | -12.4 |
| 5-day adj per customer | 2006 | 16,252,404 | 0.7% | 0.1% | -7.8 |
| high 3 of 5 | 2006 | 20,691,963 | -1.1% | -1.0% | -10.9 |
| 5-day adj per event (AMI illustrative ) | 2006 | 21,009,514 | 0.4% | -2.8% | -10.3 |
| high 3 of 6 | 2006 | 21,598,233 | -5.0% | -2.1% | -13.2 |

Comparison of Top 4 Residential CRL performances

| Residential Reference Level | Year | Percent Able to earn rebate | Percent Disadvantaged |
| --- | --- | --- | --- |
| High 3 of 5 | 2004 | 80.78 | 19.22 |
| High 3 of 6 | 2004 | 81.49 | 18.51 |
| 5-day adj per customer | 2004 | 81.60 | 18.40 |
| 5-day adj per event | 2004 | 80.76 | 19.24 |
| High 3 of 5 | 2005 | 76.55 | 23.45 |
| High 3 of 6 | 2005 | 78.57 | 21.43 |
| 5-day adj per customer | 2005 | 77.81 | 22.19 |
| 5-day adj per event | 2005 | 77.22 | 22.78 |
| High 3 of 5 | 2006 | 81.79 | 18.21 |
| High 3 of 6 | 2006 | 86.37 | 13.63 |
| 5-day adj per customer | 2006 | 84.19 | 15.81 |
| 5-day adj per event | 2006 | 80.27 | 19.73 |

| Small Commercial Reference Level | Year | Total error | Average Error | Median Error | Total Rebate No DR in Millions $ |
| --- | --- | --- | --- | --- | --- |
| 5-day adj per cust | 2004 | 3646237 | -0.7% | -0.1% | -1.9 |
| high 3 of 10 | 2004 | 4617309 | -0.2% | -0.4% | -2.3 |
| high 4 of 10 | 2004 | 4689384 | 2.9% | 0.0% | -1.9 |
| 5-day adj per event | 2004 | 4919736 | -2.7% | -2.2% | -2.9 |

SDG&E Response 6-Continued-Continued:

| 5-day adj per cust | 2005 | 3944897 | -0.2% | 0.0% | -2.0 |
| --- | --- | --- | --- | --- | --- |
| high 3 of 10 | 2005 | 4250389 | -1.5% | -0.6% | -2.4 |
| high 4 of 10 | 2005 | 4256797 | 1.1% | -0.1% | -1.9 |
| 5-day adj per event | 2005 | 5012573 | -2.4% | -2.3% | -2.9 |
| 5-day adj per cust | 2006 | 3532453 | -0.7% | 0.0% | -1.9 |
| high 3 of 10 | 2006 | 4216545 | -1.7% | -1.0% | -2.4 |
| high 4 of 10 | 2006 | 4581816 | -6.2% | -2.2% | -3.4 |
| 5-day adj per event | 2006 | 4924041 | -8.4% | -3.1% | -4.0 |

| SmCom Reference Level | Year | Percent Able to Earn Rebate | Percent Disadvantaged |
| --- | --- | --- | --- |
| High 3 of 10 | 2004 | 91 | 9 |
| High 4 of 10 | 2004 | 88 | 12 |
| 5-day adj pe | 2004 | 90 | 10 |
| 5-day adj pe | 2004 | 91 | 9 |
| High 3 of 10 | 2005 | 96 | 4 |
| High 4 of 10 | 2005 | 93 | 7 |
| 5-day adj pe | 2005 | 88 | 12 |
| 5-day adj pe | 2005 | 86 | 14 |
| High 3 of 10 | 2006 | 95 | 5 |
| High 4 of 10 | 2006 | 94 | 6 |
| 5-day adj pe | 2006 | 89 | 11 |
| 5-day adj pe | 2006 | 88 | 12 |

Reference Level Specific Definitions

SDG&E Response 6-Continued-Continued:

| 5-day adj per cust (Residential) | Takes the average hourly on-peak usage of the previous 5 similar days and adjusted this amount by adding the difference between the customers’ average hourly semi-peak use during the event day and the average hourly semi-peak use during the 5 similar days. Semi-peak consists of the hour before and 2 hours after the event. The adjustment is different for each individual customer and therefore could be gamed if the customer increased their semi-peak use on the event day. |
| --- | --- |
| 5-day adj per cust (Small Commercial) | Same as residential except the semi-peak period is two hours before the event |
| 5-day ad per event residential | The average of the 5- previous similar days is taken and multiplied by a fixed percent to account for temperature. The percent increase is the same for every customer but different for each event. The percent increase is determined by taking the total residential semi-peak load on the event day and comparing it to the total residential semi-peak load on the 5 similar days. Semi-peak period is one hour before and two hours after the event. |
| 5-day adj per event small commercial | Same as residential per event except the semi-peak period is the two hours before the event and the total small commercial load instead of the total residential load is used to calculate the adjustment. |

Response Prepared By:  Leslie Willoughby

Question 7: Referring to the same testimony, on page LW-9:

- Please provide the source for the statement on lines 7-9, e.g. the SPP reports.

- Did the high responders have any automated demand response technology?

- What is the percentage of the high responders of the total number of SPP participants?

- Based on SDG&E’s proposed CRL method, what percent of residential customers have a disadvantage error greater than 29%?

- Did SDG&E perform any analysis to support the statements on lines 3 to 7?  If it did, please provide such analysis.  Has SDG&E considered factors that might affect customer’s peak usage such as temperature variation?  If it has, please provide the analysis.  Also, explain whether this statement from lines 4 to 7 is still true if the temperature for the PTR event day is higher than the five previous days and why.

- Provide the temperature data from 2004 to 2006 for the nine annual PTR event days and the five previous like days.

SDG&E Response 7:

- The source for lines 7-9 was an analysis of the SPP data performed for SDG&E by Charles Rivers Associates in the summer of 2005.  A summary of the results is below:

In order to identify the distribution of demand response across the population of consumers, we analyzed the SPP data for statewide climate zones 2 and 3 for customers on the CPP-F rate treatment. We estimated energy demand models functionally similar to those underlying the SPP PRISM models for each customer in the database.  The customer-specific price elasticities were then used to predict the change in peak-period energy use for each customer under the CPP-F tariff. The figure below shows the cumulative contribution of customers toward aggregate demand response. As seen, 30 percent of customers provide 80 percent of the demand-response.

SDG&E Response 7 - Continued:

We then defined a high-responder as a customer that falls within this top 30 percent of responders and a low-responder as one of the remaining 70 percent of customers. A comparison of high and low responders shows the following:

- Average monthly energy use is about 70 percent greater for high responders (775 kWh) than for low responders (453 kWh)

- High responders have a 66 percent saturation of central air conditioning, compared with only 29 percent for low responders.

- More high responders have pools and spas, higher incomes, are more highly educated, and live in larger, single family homes than do low responders

- The average reduction in peak-period energy use on critical days for high responders is 29 percent, compared with only a 6.5 percent reduction for low responders

The average elasticity of substitution for high responders is more than six times larger than the average for low responders.

- No, the high responders were not offered any enabling technology.

- The high responder analysis only included SPP track A CPP-F customers in climate zones 2 and 3 because San Diego had no customers in the SPP study that were in climate zones 1 and 4. The high responders were 30% of the customers in this portion of the SPP.

| Reference Level | Year | disadvantage (Error > 29%) |
| --- | --- | --- |
| High 3 of 5 | 2004 | 12% |
| High 3 of 5 | 2005 | 14% |
| High 3 of 5 | 2006 | 6% |

- No.  The statement on lines 3 to 7 only states that if the a residential customer uses the same amount of energy during the on-peak period on the PTR event day as they used on the lowest 2 of the previous 5 similar days then that customer will earn a rebate.  This is true based on the definition of the reference level and does not require any analysis.

SDG&E Response 7 - Continued:

- Daily maximum temperatures from San Diego’s Miramar weather station are in the file below:

Response Prepared By:  Leslie Willoughby

- Question 8: Referring to Mr. Magill’s Testimony, page 13,

  - What was the cost recovery mechanism for SDG&E’s 20/20 program for residential customers?  Is this program currently available in SDG&E’s service territory?

  - What are the adopted cost recovery mechanisms for SDG&E’s current demand response programs for all customer classes?

  - Is SDG&E proposing PTR as a demand response program or as a default dynamic rate design for residential customers?

  - Is SDG&E’s proposing a different cost recovery mechanism for PTR from its current demand response programs?  If it is, please explain the reason(s) why the Commission should treat PTR differently from other demand response programs.

- SDG&E Response 8:

- Costs such as administration, customer outreach and communications associated with the Residential 20/20 program were recorded in SDG&E’s Advanced Metering and Demand Response Memorandum Account (AMDRMA) on a monthly basis.  On an annual basis, these costs were transferred to SDG&E’s Rewards and Penalties balancing Account (RPBA) and recovered through SDG&E’s distribution component.  Bill credits for energy reductions associated with the Residential 20/20 program were recorded and recovered through SDG&E’s Energy Resource Recovery Account (ERRA).  The Residential 20/20 program is no longer available in SDG&E’s service territory.

- Costs such as administration, customer outreach and communications associated with SDG&E’s demand response programs are recorded in SDG&E’s Advanced Metering and Demand Response Memorandum Account (AMDRMA) on a monthly basis.  On an annual basis, these costs are transferred to SDG&E’s Rewards and Penalties balancing Account (RPBA) and recovered through SDG&E’s distribution component.  Bill credits for energy reductions associated with the Commercial and Industrial 20/20 program are recorded and recovered through SDG&E’s Energy Resource Recovery Account (ERRA).

- SDG&E Response 8 - continued:

- The Peak Time Rebate (PTR) is neither a demand response program nor default dynamic rate.  The PTR is an incentive rate that is designed to encourage residential customers to reduce demand during the highest system demand days. Traditional programs or default rates usually require customers to either enroll or opt-out to receive potential benefits or avoid potential risks.  Customers are automatically eligible for the PTR credit once an AMI meter is installed.  Customers are not required to enroll to receive benefits and there is no impact on the customer’s otherwise applicable bill if they choose not to participate.

- No.  Specifically, PTR credits provided to customers will be recovered through the ERRA.  Customer communications, education, outreach, implementation and administration will be recovered through the distribution rate component (AMI balancing account).

- Response Prepared By:  James Magill

Question 9: Please provide the total number of residential customers for each climate zone.  Within each climate zone, provide: 1) the number of customers, 2) total energy usage, and 3) peak usage for each usage group (Tiers 1 to 5).  Also, please provide the total summer and winter tier usages for each climate zone.  The requested data should be based on SDG&E’s 2006 record or most recent forecast.  Please provide the data in the following formats.

SDG&E Response 9:

Response Prepared By:   Leslie Willoughby