QUESTION 1:

The following questions pertain to differences in SoCalGas costs between the 2009 BCAP (A.08-02-001) and the current application. The questions are based on data found in the workpaper “SCG 2013TCAP LRMC Customer Costs.xls” from the current application and the workpaper “Smith SCG LRMC model – errata 10-6-08 final.xls” from the 2009 BCAP application.

  - Please explain why Long Beach is allocated $3,430 in customer service O&M costs in the current application (tab “cust MC,” cell Y28), while Long Beach was allocated $0 in customer service O&M costs in the prior BCAP (tab “cust MC,” cell Y28). Please provide documentation supporting the 2013 customer service O&M cost for Long Beach.

  - Please explain the reason for the increase in cost for Meter, Reg. Investment for transmission-level G-30 customers from $126,778 (2009$) per customer (tab “cust 5,” cell R11) to $253,327 (2010$) per customer (tab “cust 5,” cell R10). In your response, please explain why the cost increase is proportionately greater for G-30 customers than for other customers, and please provide supporting documentation.

  - Please explain the reason for the significant increases in replacement costs for Turbine MSAs and Orifice Meters since the 2009 BCAP, as summarized in Table 1 below.  Please include in your response a breakdown of the increase by cost component, using the cost components shown in the 2013 BCAP workpapers on tab “Wholesale Meter Costs,” cells C10:C16, and please provide supporting documentation.

Table 1: Replacement Cost for Turbine MSAs and Orifice Meters

|  | 2009 BCAP (2009$) | 2013 BCAP (2010$) | % Change |
| --- | --- | --- | --- |
| Turbine MSAs | 88,615 | 617,840 | 597% |
| Orifice Meters | 173,675 | 1,284,167 | 639% |

Source: Tab “cust 6,” cells G22:G23 in the 2009 BCAP workpapers and cells G24:G25 in the 2013 BCAP workpapers

  - Please explain the reason for the significant increases in replacement costs for GEMS, meters, and regulators for wholesale customers since the 2009 BCAP, as summarized in Table 2 below. Please include in your response the number of each type of meter used to assess these costs for each wholesale customer in the 2009 BCAP (comparable to the data provided in the 2013 BCAP workpapers, tab “Wholesale Meter Costs,” cells B26:B59) and the reason for any changes to these meter profiles. Please provide supporting documentation.

Table 2: Replacement Cost for GEMS, Meter, Regs for Wholesale Customers

|  | 2009 BCAP (2009$) | 2013 BCAP (2010$) | % Change |
| --- | --- | --- | --- |
| LB | 243,392 | 5,165,165 | 2,022% |
| SDG&E | 189,380 | 12,175,338 | 6,329% |
| SWG | 76,184 | 3,792,235 | 4,878% |
| Vernon | 16,484 | 2,568,333 | 15,481% |
| DGN | 152,367 | 617,840 | 305% |

Source: Tab “cust 7” of the 2009 and 2013 BCAP workpapers

  - Please explain the reason for the significant increase in Customer Services O&M costs for large EG customers from $1,490 (2007$) in the 2009 BCAP (tab “cust 8,” cell V11) to $29,399 (2010$) in the 2013 BCAP (tab “cust 8,” cell V14). In your response, please explain why the cost increase is proportionately greater for large EG customers than for other customers, and please provide supporting documentation.

  - Please explain the reason for the greater reduction in O&M costs for Meters, Reg & MSAs for residential and non-residential core customers than for most non-core customers, as shown in Table 3 below. Please provide supporting documentation.

Table 3: O&M Cost for Meters, Reg & MSAs

|  | 2009 BCAP (2007$) | 2013 BCAP (2010$) | % Change |
| --- | --- | --- | --- |
| Residential | 13,230 | 7,577 | -43% |
| Core Non-Residential | 1,562 | 1,085 | -31% |
| G-30 | 2,897 | 2,596 | -10% |
| Small EG | 640 | 580 | -9% |
| EG G-50 | 171 | 126 | -26% |
| EOR | 109 | 84 | -23% |
| Long Beach | 17 | 18 | 6% |
| SDG&E | 18 | 18 | 0% |
| Southwest Gas | 34 | 36 | 6% |

Source: Tab “cust 8,” row 27 in the 2009 BCAP workpapers and row 30 in the 2013 BCAP workpapers

RESPONSE 1:

1a:  Please see cell O95 on the “LRMC Summary” tab of the attached file for the source of the $3,430 allocated to Long Beach for Customer Services costs in this TCAP.  This cost is comprised of Field Services work, which is allocated to the classes based on work hours recorded.  A small number of hours were identified as being related to Long Beach.  The $3,430 in question impacts the rate for Long Beach by less than $0.00001/therm, so removing the allocation would not impact the tariffed rate.

1b:  The attached file compares the 2009 BCAP meter cost data against the 2013 meter cost data.  While the G-30 transmission costs did increase the most, there were other classes (e.g. single-family residential and very large G-10) that had similarly large increases in costs.  While all classes experienced increases in labor costs, the classes with the largest meter cost increases experienced commensurately large labor increases as well.  Also, each class’ average meter cost is based on a weighted average of the various meter sizes within it.  The meter count used to develop the class-average meter cost for the G-30 transmission customer class increased from the 2009 BCAP to the 2013 TCAP.  This increase was concentrated at the largest meter size segment and pulled up the weighted-average of the class.

1c:  The table below summarizes the difference between the amounts used in the 2009 BCAP filing and the current TCAP filling. Increases were due to Labor, Contract Costs, and Materials.  SoCalGas switched from Orifice Meters to Ultrasonic meters, and the costs of those meters has increased.

|  | Turbine Meters | Turbine Meters |  |  |
| --- | --- | --- | --- | --- |
|  | 2009 BCAP | 2013 TCAP | $ change | % change |
| Avg. Meter Cost | $11,274 | $17,118 | $5,844 | 52% |
| Avg. Labor Cost | $9,848 | $121,021 | $111,173 | 1129% |
| Avg. Contract Cost | $24,190 | $242,000 | $217,810 | 900% |
| Materials | $28,016 | $227,203 | $199,187 | 711% |
| Regulator Cost | $3,938 | $2,303 | ($1,635) | -42% |
| GEMS Device Cost | $11,350 | $8,195 | ($3,155) | -28% |
| TOTAL | $88,616 | $617,840 | $529,224 | 597% |
|  | Orifice Meters | Ultrasonic Meters |  |  |
|  | 2009 BCAP | 2013 TCAP | $ change | % change |
| Avg. Meter Cost | $32,400 | $119,284 | $86,884 | 268% |
| Avg. Labor Cost | $18,625 | $204,372 | $185,747 | 997% |
| Avg. Contract Cost | $35,000 | $379,500 | $344,500 | 984% |
| Materials | $72,817 | $561,130 | $488,313 | 671% |
| Regulator Cost | $5,150 | $0 | ($5,150) | -100% |
| GEMS Device Cost | $9,683 | $19,881 | $10,198 | 105% |
| TOTAL | $173,675 | $1,284,167 | $1,110,492 | 639% |

1d: The gross costs listed in Table 2 increased partially because the per meter costs increased as discussed in Response 1c.  Additionally, the number of meters used to determine the gross costs was updated to be based on a current listing of meters at the various wholesale customers.  The table below compares the number and type of meters used in the 2009 BCAP and the 2013 TCAP.

1e:  This cost is comprised of Field Services work, which is allocated to the classes based on actual work hours recorded for each rate class. See workpapers provided in Response 1.a

1f:  The O&M costs shown in Table 3 are almost entirely related to FERC Account 893, Maintenance of Meters and House Regulators, and costs for overhead accounts.  In total, the O&M costs decreased from $18.6 million to $12.1 million.  However, the Wholesale customer classes are only directly allocated costs from subaccount 893.100.  This subaccount increased slightly, from $1,079k to $1,154k.  So while overheads decreased from 2009 to 2013, which brought down wholesale O&M costs, subaccount 893.100’s increased costs netted a slight increase for wholesalers, while core’s allocated costs decreased.  See attached files for derivations of O&M costs for 2009 and 2013.

QUESTION 2:

The following question refers to the SoCalGas NCO analysis in workpaper “SCG 2013TCAP LRMC Customer Costs.xls, tab “cust MC.”

According to the comment in cell C81, the data in Row 81 (“New Hookups Rate”) is based on the number of new customers in 2010. Please provide the number of new customers in each customer class and the total number of customers in each customer class for each of the last five years for which these data are available.

RESPONSE 2:

QUESTION 3:

The following questions refer to workpaper “SCG 2013TCAP LRMC Customer Costs.xls, tab “2010 RECC.”

  - Please provide a working Excel spreadsheet showing the calculation of the RECC and PVCC factors in Columns W and X.

  - Please provide the formula for calculating the RECC Factors, as used in Column W.

  - Please provide the formula for calculating the PVCC Factors, as used in Column X.

  - Please explain why Meter Installations have a different book life than Meters (cells F33 and F34). Please include in your response an explanation of what items are recorded to each of these accounts.

  - For each year within the most recent five-year period for which these data are available, please provide the number of replacements associated with FERC Account G-380 (“Services”) and the number of assets included in this account. Please provide supporting documentation.

  - For each year within the most recent five-year period for which these data are available, please provide the number of replacements associated with FERC Account G-381 (“Meters”) and the number of assets included in this account. Please provide supporting documentation.

  - For each year within the most recent five-year period for which these data are available, please provide the number of replacements associated with FERC Account G-382 (“Meters Installations”) and the number of assets included in this account. Please provide supporting documentation.

  - For each year within the most recent five-year period for which these data are available, please provide the number of replacements associated with FERC Account G-383 (“House Regulators”) and the number of assets included in this account. Please provide supporting documentation.

RESPONSE 3:

- SoCalGas objects to this request on the grounds that it requests confidential and proprietary trade secret information. Pursuant to Commission Rule of Practice and Procedure 10.4, SoCalGas and SDG&E elect to make model runs for intervenors rather than releasing the model.  The formula for calculating the RECC and PVCC factors are provided in Response 3b and 3c, respectively.

- The RECC calculation employs a standard formula used by electric and gas utilities.  Each RECC factor is calculated in the following manner:

Where,

- PVRR is the present value of the revenue requirements associated with a particular capital asset.  The revenue requirements are the calculated annual stream of capital carrying costs spanning the life of the asset.  Capital carrying costs include:

  - Book depreciation (return of capital)

  - Salvage

  - Authorized rate of return on equity and debt (return on capital)

  - Income taxes

  - Property taxes

- ROR is the discount rate, or authorized rate of return.

- Inflation is the expected rate of inflation over the life of the asset.

- Book life is the asset’s book life in years.

- The PVCC factors are essentially the present value multiplier for a stream of revenue requirements associated with a particular asset type.  The formula can be written as:

PVCC =

Where,

- Cn is the revenue requirements associated with a particular capital asset in year n.  The revenue requirements are the calculated annual stream of capital carrying costs spanning the life of the asset.  Capital carrying costs include:

  - Book depreciation (return of capital)

  - Salvage

  - Authorized rate of return on equity and debt (return on capital)

  - Income taxes

  - Property taxes

- ROR is the discount rate, or authorized rate of return.

- SoCalGas’ book lives are authorized by the CPUC during the company’s general rate case proceedings and are not being proposed in this Application. 

However, the Meters and Meter Installation accounts are two separate and distinct accounts with different accounting activities as defined in the FERC Uniform Systems of Accounts.  As a result, the activities drive the variation in book life.  For example, meters are capitalized when purchased whereas installation could occur in a later period.  Additionally, all re-manufacturing costs (labor and non-labor) are capitalized to the Meters account when such work extends the life of the meter.  For the Meter Installations account, only new meter installations or first-time installations are capitalized.

Items recorded to the Meters account include meters, regulators, gauges, and all costs (labor and non-labor) that extend the life of the meter.  Items recorded to the Meter Installations account include first-time installation costs (labor and material costs incurred during installation) for rotary, turbine, GEMS, orifice meters and first time installation of a meter guard.

- Account 380, Services

| FERC Account # | Account Description | Year | # of Replacements per Year |  |
| --- | --- | --- | --- | --- |
| 380 | Services |  |  |  |
|  |  | Year 2007 | 13,944 |  |
|  |  | Year 2008 | 12,528 |  |
|  |  | Year 2009 | 12,725 |  |
|  |  | Year 2010 | 11,891 |  |
|  |  | Year 2011 | 12,278 |  |

SoCalGas does not understand what is meant in the question by “number of assets included in this account”.

- Account 381, Meters

| FERC Account # | Account Description | Year | # of Replacements per Year |  |
| --- | --- | --- | --- | --- |
| 381 | Meters |  |  |  |
|  |  | Year 2007 | 165,680 |  |
|  |  | Year 2008 | 166,648 |  |
|  |  | Year 2009 | 171,993 |  |
|  |  | Year 2010 | 177,387 |  |
|  |  | Year 2011 | 148,463 |  |

SoCalGas does not understand what is meant in the question by “number of assets included in this account”.

- Account 382, Meters Installations

| FERC Account # | Account Description | Year | # of Replacements per Year |  |
| --- | --- | --- | --- | --- |
| 382 | Meter Installations |  |  |  |
|  |  | Year 2007 | 165,680 |  |
|  |  | Year 2008 | 166,648 |  |
|  |  | Year 2009 | 171,993 |  |
|  |  | Year 2010 | 177,387 |  |
|  |  | Year 2011 | 148,463 |  |

SoCalGas does not understand what is meant in the question by “number of assets included in this account”.

- Account 383, House Regulators

| FERC Account # | Account Description | Year | # of Replacements per Year |  |
| --- | --- | --- | --- | --- |
| 383 | House Regulators |  |  |  |
|  |  | Year 2007 | 52,827 |  |
|  |  | Year 2008 | 40,172 |  |
|  |  | Year 2009 | 66,562 |  |
|  |  | Year 2010 | 65,137 |  |
|  |  | Year 2011 | 42,168 |  |

SoCalGas does not understand what is meant in the question by “number of assets included in this account”.

QUESTION 4:

The following questions refer to the workpaper “SCG 2013TCAP LRMC Customer Costs.xls, tab “cust 5.”

- Please state whether the Investment per Customer in Service Lines shown in row 26 includes any costs for replacing existing service lines.

- Please state whether the Investment per Customer in Service Lines shown in row 26 refers to the investment per new customer or the investment per total number of customers in each customer class. Please provide the source for these data.

- Please state whether the Investment per Customer in Service Line Replacements shown in row 42 refers to the investment per new customer or the investment per total number of customers in each customer class. Please provide the source for these data.

- Please explain why the per-customer investment for Service Lines Replacement Costs (row 42) greatly exceeds the per-customer investment for Service Lines (row 26). Please provide supporting documentation.

RESPONSE 4:

- No.

- Row 26 refers to the average cost per new customer in each customer class.  The attached workpaper details the derivation of the Service Line investment per customer by customer class.

- Row 42 refers to the average cost to replace one customer’s service line in each customer class. See the workpaper provided in response 4-b.

- Construction for New Installations have lower costs due to less issues arising, as compared to the replacement of an existing service, from various obstructions such as walkways, driveways, landscaping, size of work area, etc. which are found with replacements.