Small Customer Technology Deployment (SCTD) Load Impact Report

April 1st 2011

I. Program Description

SDG&E’s Small Customer Technology Deployment (SCTD) Program will offer

automated DR enabling technologies at no cost for up to 15,000 participating SDG&E residential customers and as many as 3,000 small commercial customers (<100 kW). SDG&E proposes using Smart Meter interval data to identify, market to, and install load control devices in the homes of residential and small commercial businesses with significant air conditioning and residential customers with mid-day pool pump usage.

SDG&E’s SCTD program will target customers that participate in the Whole House and

Small Commercial Direct Install programs that SDG&E offers as part of our energy efficiency portfolio. Additionally SDG&E will explore opportunities to target the roughly 200 participants from our Borrego Springs Micro Grid Comprehensive Energy Efficiency Delivery Pilot. These customers have demonstrated a desire for energy management and will be ideal candidates for a more comprehensive IDSM solution. These automated enabling technologies will provide incremental load reduction benefits during demand response events and create a technology platform that will support future dynamic pricing rate design for residential and small commercial customers. Potential end use loads include central air conditioning, refrigeration, lighting, pool pumps and electric water heaters. SDG&E may consider 3rd party Aggregators or vendors for possible implementation of the SCTD program including the recruitment of potential customers. Although commercially available enabling technologies for the residential and commercial markets exist, the installation complexities require the average residential customer to have an experienced vendor to make sure the devices are installed and commissioned to the customer’s smart meter properly. In addition to providing deeper load reductions, the SCTD program is designed to increase the number of vendors capable of these installations and, over time SDG&E believes that the SCTD program will influence technology solutions that are simple enough for the average home owner or renter to install and utilize during a DR event.

II. Forecast Summary

The SCTD forecast accounts for load impacts from residential PCT and pool pumps and from small commercial customer with enabling technology. The load impacts for residential PCT were calculated by the consulting firm Freeman, Sullivan and company (FSC). The load impacts for the residential pool pumps and small commercial load impacts were calculated by SDG&E.  Table 1 below contains the forecast for 2012-2014 for the entire SCTD program. The details of each part of the program forecast will be presented in section III and IV of this report.

| Table1: Small Customer Technology Deployment forecast (MW) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Year | Weather | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| Small Customer Technology Deployment | 2012 | 1in 2 | 1 | 1 | 1 | 3 | 5 | 4 | 5 | 6 | 6 | 6 | 1 | 1 |
| Small Customer Technology Deployment | 2013 | 1in 2 | 2 | 2 | 2 | 5 | 9 | 6 | 9 | 10 | 10 | 10 | 2 | 2 |
| Small Customer Technology Deployment | 2014 | 1in 2 | 2 | 2 | 2 | 6 | 11 | 8 | 11 | 12 | 13 | 12 | 3 | 2 |

- Residential Load Impact forecast

Previous SDG&E Smart Thermostat studies and Summer Saver studies have shown that one factor that decreases the load impacts and cost-effectiveness of these programs is that customers who never or seldom use their air-conditioners join the program and receive an incentive.  The SCTD program improves this issue in two ways.  One way this issue is improved is that no flat incentive is provided.  Only a PTR incentive is provided and a PTR incentive is only paid if a customer’s usage is lower than their customer reference level.  The other improvement this program makes is that Smart Meter data will be used to market the program to customers likely to have high on-peak air-conditioner usage.  Hourly Smart Meter whole house data can be used to identify customers who are likely use their air-conditioner on-peak. In order to estimate the effects that targeting customers using hourly whole house smart meter data will have on the load impacts Freeman Sullivan and Company (“FSC”) conducted analysis on behalf of SDG&E using the load data from the load research air-conditioning sample.  This sample is a randomly selected sample of customers with central air-conditioning.  These customers have a meter both on their home and on their air-conditioner. Using the whole house data only, FSC ran a regression model and identified the top 35% of customer most likely to have high on-peak air-conditioning usage.  The air-conditioning usage of these top customers was then used to create the reference load for the residential Programmable Communicating Thermostat (“PCT”) forecast.

The percentage load impacts for residential PCT program are incremental to the PTR percentage load impacts for the 50% of customers who are aware of PTR events. For the 50% of customers unaware of PTR events the full percentage load reduction achieved by PCT was used.  The full percentage load reduction used for the forecast comes from the SDG&E Smart Thermostat studies.  The incremental load reduction above and beyond the PTR rate was informed by the Connecticut “Plan it Wise” pilot.  This pilot offered both a PTR rate and a PTR rate with enabling technology to customers.  Based on the results of this pilot a 16% incremental impact of enabling technology was assumed for the 50% of customers being aware of PTR events.  The 16% is the percentage of air-conditioning load reduced, not the percentage of whole house load reduced.

The full report and detailed ex-ante tables provided by FSC are attached below.

Hourly load impacts per customer for the pool pumps were taken from Table 21 page 31 of the Pool Pump Demand Response Potential Study. This study was prepared by the design and engineering services customer service business unit at SCE and published in June of 2008. The forecast assumes that pool pump load impacts do not change with weather. The forecasted number of residential customers enrolled in the pilot is 7,500 by 2012, 12,500 by 2013 and 15,000 by 2014. One-third of participants are forecasted to accept pool pump technology.

- Small Commercial Load Impact Forecast

The small customer technology deployment program will also provide enabling technology to small commercial customers enrolled on the Peak Shift at Home rate.  The reference load for the small commercial forecast is based on SDG&E dynamic load profile hourly small commercial customer load shape.  A regression analysis using weekday data was run with day of the week and weather as explanatory variables. This model was used to predict the reference load for the 1 in 2 and 1 in 10 standard temperatures used for all program forecast. Since the Statewide Pricing Pilot small commercial update results showed no statically significant load reduction in response to the CPP rate alone an incremental load impact forecast is not necessary.  The percentage load impact in response to enabling technology used in the forecast is 19.3% consistent with the 2009 SDG&E CPP-D Auto-DR M&E results.  The forecast assumes that 1,000 customers enroll by 2012, 2,000 by 2013 and 3,000 by 2014.

California Statewide Pricing Pilot Commercial and Industrial Analysis Update June 28 2006 CRA International