QUESTIONS 1-3 FOR SDG&E

QUESTION 1:

Please provide the recorded cost and number of replacement services, by Maximum Meter Flow Range, for each of the last five years.  Please disaggregate costs by civil, materials and structures and labor.

RESPONSE 1:

SDG&E objects to this question on the grounds that it is unreasonably burdensome and requests information not relevant to this proceeding,  Without waiving these objections, and subject thereto, SDG&E responds as follows:

The information being requested is not readily available to the cost allocation witnesses because historical recorded replacement data was not used to develop their testimony. Instead, estimates of replacement activity were used in the New Customer Only with Replacement Cost Adders method of allocating customer costs. Those estimates were done using the inverse of the accounting book lives of the assets and were included in the workpapers.

The information that SDG&E can provide within the time frame proposed by TURN is as follows:  total service line costs for each of the last five years (new business and replacements combined) and total number of feet (which was converted to number of services), disaggregated by service size.

The attached file has the specified information by size of pipe, total feet, and conversion to number of services.

The table below is a summary of total costs and number of service line jobs including replacements and additions from years 2007-2011 is as follows:

| Year | Number of Services | Total Cost |
| --- | --- | --- |
| 2011 | 2,811 | $3,310,373 |
| 2010 | 3,125 | $3,173,627 |
| 2009 | 3,321 | $3,969,584 |
| 2008 | 4,365 | $2,425,813 |
| 2007 | 4,895 | $3,954,081 |

QUESTION 2:

Please provide the number of active accounts, by Maximum Meter Flow Range, for each of the last five years.

RESPONSE 2:

The attached file contains the requested information.  Since 2011 data was not available, 2006 was provided instead in order to include 5 years of data.

Only meter counts for 2010 were utilized in the TCAP proposed Rental method, and in the New Customer Only with Replacement Cost Adder method. No historical meter counts beyond that were used or provided in workpapers.

QUESTION 3:

Please provide the cost of and number of meter/regulator sets replaced, by Maximum Meter Flow Range, in each of the past five years.

  - Does installation of SmartMeter modules/meters affect the meter replacement rate in SCG in any of the years from 2008-2011.  If so, how many additional or fewer meters are replaced due to this effort? How does it affect the forecast of meter replacements?

RESPONSE 3:

SDG&E objects to this question on the grounds that it is unreasonably burdensome and requests information not relevant to this proceeding.  Without waiving these objections, and subject thereto, SDG&E responds as follows:

The information being requested is not readily available to the cost allocation witnesses because historical recorded replacement data was not used to develop their testimony. Instead, estimates of replacement activity were used in the New Customer Only with Replacement Cost Adders method of allocating customer costs. Those estimates were done using the inverse of the accounting book lives of the assets and were included in the workpapers.  Attempts to locate the data requested by TURN were not successful due to meter and regulator replacement data not being recorded in a manner which allows for a database query.

- SDG&E found some meters that needed replacement during the SmartMeter installation process. These meters were replaced when the index mounting screws on the existing meter failing during installation, rendering the meter inoperable and necessitating the replacement of the meter.  This issue created a one-time impact to the replacement rate of meters and will not affect future replacement rates.  The number of meters replaced in the SmartMeter roll-out amounted to approximately 3% of the meter population over 800,000.

QUESTIONS 4-5 FOR SOCALGAS

QUESTION 4:

Please explain the difference between “periodic Meter Change Sets” and “Routine Meter Change Sets”.  Which of these measures, if either, most closely relates to the meter/regulator replacements represented by “Weighted Replacement Factor for Meter and House Reg”, line 116 in “cust MC” in SCG 2013 TRCAP LRMC Customer Costs?

RESPONSE 4:

Periodic meter changes are changes that are planned by the meter department.  Routine meter changes are unplanned changes that occur when a customer identifies a potential issue and it results in a meter change.

Neither of these measures relate to the meter replacement factors used in the TCAP workpapers.  The “Weighted Replacement Factor(s) for Meter and House Reg(s)” found on the “cust MC” tab are developed on the “cust 10” tab.  They are based on the average of the inverse of the book lives for Installations, Meters, and Regulators, weighted by the cost of each of those components.  They are not based on any recorded periodic or routine meter changes.

QUESTION 5:

Does installation of SmartMeter modules/meters affect the meter replacement rate in SCG in any of the years from 2008-2011.  If so, how many additional or fewer meters are replaced due to this effort? How does it affect the forecast of meter replacements?

RESPONSE 5:

No, the Advanced Meter Infrastructure (AMI) program at SoCalGas did not impact the meter replacement rate in the years 2008 to 2011.

QUESTIONS 6-7 FOR SDG&E & SOCALGAS

QUESTION 6:

The response to TURN DR-3, question 1 states, in part:  “According to the business unit, the driving factor for the service line increase was an increase in the trenching costs required for replacing a service line.”

  - TURN’s question asked for an explanation of the increases in replacement costs for meters and regulators, as well as service lines.  With the understanding that these costs “were contained within 15-30%” please explain the increases between the 2009 figures and the 2013 figures.

  - Was there any factor other than an increase in the trenching costs required for replacing a service line that contributed to the increase for the service line increase?  If so, please identify each such factor and explain how and approximately how much it contributed to the increase.

  - For the increase in trenching costs, please provide an explanation of each utility’s understanding of why the costs of trenching have increased in recent years, and provide any recorded data demonstrating such increases, as well as analysis the utility has performed of that trenching cost increase.

d.	TURN is assuming that the response to question DR 3-1 provided for SD&GE would have been generally the same for SoCalGas.  If that is not the case, please provide a response to question DR 3-1 for SoCalGas.

RESPONSE 6:

- At SDG&E, costs for the meter set assembly (MSA) have increased in the range of 15 to 30% from the 2009 BCAP to the 2013 TCAP due predominantly to the fact that the Moving Average Price (MAP) for all components in the MSA (pipe, fittings, elbows, filters, etc.) have increased since the time of the 2009 BCAP analysis. The MAP is tracked by our Logistics department for all the parts and components used in the company. These are cost increases primarily attributed to inflation of material prices and market price fluctuations.

- For gas service replacements at SDG&E, the principal cost drivers resulting in the difference between costs presented for the 2009 TCAP and the 2013 TCAP were the labor, material, equipment, and environmental costs associated with trenching for the replacement service. Other factors that contributed to a lesser degree were increases in construction overheads and escalation.

Expenses for labor, material, equipment, and environmental costs are not recorded in a format that would allow isolation of these cost elements individually for analysis.  However, the increases in the construction overheads can be identified.  The construction overheads used were 1.4 times the material costs in 2009 BCAP filing versus 1.6 times the material costs in the 2013 TCAP filing.

- The contributing factors associated with service replacement trenching that resulted in cost increases include but are not limited to the following:

- More restrictive municipality requirements for the timing, installation and street repair associated with trench construction. These include:

- Curb-to-curb street surface repairs, not just surface of the trench replacement greatly increasing the cost for street repair.

- The requirement for specialized street coatings following trenching and street repair.

- More restrictive project construction times to avoid peak traffic hours, requiring overtime to work on weekends or after normal working hours.

- Greatly increased need for traffic control in city and county streets. This has resulted in increased contract costs for labor and materials to provide Municipality approved traffic control during construction.

- Increased material costs. This includes the costs of pipeline materials and trenching and backfill materials and the requirement for the installation of excess flow valves on each replacement residential service.

- Environmental constraints resulting in increased costs associated with contaminated soil removal, disposal and replacement.

The only recorded data readily available is the service line costs provided in Response 1. Also, the utility has not performed an analysis of that trenching cost increase.

- Generally speaking, yes.  The increases in meter costs at SoCalGas were discussed with some detail in Response 1b to Long Beach data request #5.  The factors discussed in Response 6.a-6.c above apply to both utilities.

QUESTION 7:

Please provide the number of services installed by each method (insert, bore and trench), disaggregated by rate class and service size, for each of the last five years.  Please also explain the criteria for determining which installation method to use.

RESPONSE 7:

SoCalGas and SDG&E object to this question on the grounds that it is unreasonably burdensome and requests information not relevant to this proceeding.  Without waiving these objections, and subject thereto, SoCalGas and SDG&E respond as follows:

The information being requested is not readily available to the cost allocation witnesses because it was not used to develop their testimony.  The information SDG&E can provide within the time frame proposed by TURN is as follows: total costs for each of the last five years (new business and replacements combined), by pipe diameter, and total number of feet (which was converted to number of services). See Response 1 for the total number of service lines from 2007-2011 by size of pipe for SDG&E

For SoCalGas the information that was available is shown in the attached file and includes the number of services installed by each method (insert, cut & bore, and trench) and by size of pipe for the years 2007 through 2011.  However, we do not track service pipeline by rate class, so that information is not available.

For both SDG&E and SoCalGas, the decision to insert, cut & bore, or trench in replacing a service is made on a case-by-case basis in the pre-design phase and many times modified in the construction phase as field conditions dictate. This decision depends on many factors, some of which include the reason for the replacement (corrosion, obstruction, relocation), size, type and condition of the existing service being replaced, nature of customer landscaping or surface obstructions, length of the service, and potential for conflict with other underground utilities.