Work Paper WPSDGENRCC0004

Revision 2

San Diego Gas & Electric

Energy Efficiency Engineering

Commercial Ice Machines

Measure Codes FS-D8-FSD21

# At a Glance Summary: Energy Star® Ice Machines

| Applicable Measure Codes: | FS-D8 | FS-D9 | FS-D10 | FS-D11 |
| --- | --- | --- | --- | --- |
| Measure Description: | Commercial Ice Machine, 101 – 200 lbs/day | Commercial Ice Machine, 201 – 300 lbs/day | Commercial Ice Machine, 301 – 400 lbs/day | Commercial Ice Machine, 401 – 500 lbs/day |
| Energy Impact Common Units: | Ice Machine | Ice Machine | Ice Machine | Ice Machine |
| Base Case Description: | Source: PG&E Calculations. Commercial Ice Machine, 101 – 200 lbs/day | Source: PG&E Calculations. Commercial Ice Machine, 201 – 300 lbs/day | Source: PG&E Calculations. Commercial Ice Machine, 301 – 400 lbs/day | Source: PG&E Calculations. Commercial Ice Machine, 401 – 500 lbs/day |
| Base Case Energy Consumption: | Source: PG&E Calculations. 4,503 kWh/yr | Source: PG&E Calculations. 5,550 kWh/yr | Source: PG&E Calculations. 6,946 kWh/yr | Source: PG&E Calculations. 7,878 kWh/yr |
| Measure Energy Consumption: | Source: PG&E Calculations. 4,172 kWh/yr | Source: PG&E Calculations. 4,999 kWh/yr | Source: PG&E Calculations. 6,261 kWh/yr | Source: PG&E Calculations. 7,083 kWh/yr |
| Energy Savings (Base Case – Measure) | Source: PG&E Calculations. 331 kWh/yr | Source: PG&E Calculations. 551 kWh/yr | Source: PG&E Calculations. 685 kWh/yr | Source: PG&E Calculations. 795 kWh/yr |
| Costs Common Units: | Ice Machine | Ice Machine | Ice Machine | Ice Machine |
| Base Case Equipment Cost ($/unit): | Source: PG&E Calculations. $1,976 | Source: PG&E Calculations. $2,442 | Source: PG&E Calculations. $2,459 | Source: PG&E Calculations. $2,583 |
| Measure Equipment Cost ($/unit): | Source: PG&E Calculations. $2,165 | Source: PG&E Calculations. $3,260 | Source: PG&E Calculations. $2,740 | Source: PG&E Calculations. $2,646 |
| Measure Incremental Cost ($/unit): | Source: PG&E Calculations. $189 | Source: PG&E Calculations. $818 | Source: PG&E Calculations. $281 | Source: PG&E Calculations. $63 |
| Effective Useful Life (years): | Source: DEER. EUL for Retrofit EUL for Replace on Burnout (ROB) = 12 years, and New Construction (NEW) = 12 years. | Source: DEER. EUL for Retrofit EUL for Replace on Burnout (ROB) = 12 years, and New Construction (NEW) = 12 years. | Source: DEER. EUL for Retrofit EUL for Replace on Burnout (ROB) = 12 years, and New Construction (NEW) = 12 years. | Source: DEER. EUL for Retrofit EUL for Replace on Burnout (ROB) = 12 years, and New Construction (NEW) = 12 years. |
| Program Type: | Retrofit Replace on Burnout (ROB), and New Construction (NEW). | Retrofit Replace on Burnout (ROB), and New Construction (NEW). | Retrofit Replace on Burnout (ROB), and New Construction (NEW). | Retrofit Replace on Burnout (ROB), and New Construction (NEW). |
| Net-to-Gross Ratios: | Source: 2011 DEER, Com-Default>2yrs, 0.60 | Source: 2011 DEER, Com-Default>2yrs, 0.60 | Source: 2011 DEER, Com-Default>2yrs, 0.60 | Source: 2011 DEER, Com-Default>2yrs, 0.60 |
| Important Comments: |  |  |  |  |

# At a Glance Summary: Energy Star® Ice Machines (Cont)

| Applicable Measure Codes: | FS-D12 | FS-D13 | FS-D14 |
| --- | --- | --- | --- |
| Measure Description: | Commercial Ice Machine, 501 – 1,000 lbs/day | Commercial Ice Machine, 1,001 – 1,500 lbs/day | Commercial Ice Machine, > 1,500 lbs/day |
| Energy Impact Common Units: | Ice Machine | Ice Machine | Ice Machine |
| Base Case Description: | Source: PG&E Calculations. Commercial Ice Machine, 501 – 1,000 lbs/day | Source: PG&E Calculations. Commercial Ice Machine, 1,001 – 1,500 lbs/day | Source: PG&E Calculations. Commercial Ice Machine, > 1,500 lbs/day |
| Base Case Energy Consumption: | Source: PG&E Calculations. 12,452 kWh/yr | Source: PG&E Calculations. 17,452 kWh/yr | Source: PG&E Calculations. 24,432 kWh/yr |
| Measure Energy Consumption: | Source: PG&E Calculations. 11,190 kWh/yr | Source: PG&E Calculations. 15,878 kWh/yr | Source: PG&E Calculations. 22,229 kWh/yr |
| Energy Savings (Base Case – Measure) | Source: PG&E Calculations. 1,263 kWh/yr | Source: PG&E Calculations. 1,574 kWh/yr | Source: PG&E Calculations. 2,204 kWh/yr |
| Costs Common Units: | Ice Machine | Ice Machine | Ice Machine |
| Base Case Equipment Cost ($/unit): | Source: PG&E Calculations. $3,494 | Source: PG&E Calculations. $4,751 | Source: PG&E Calculations. $6,802 |
| Measure Equipment Cost ($/unit): | Source: PG&E Calculations. $3,728 | Source: PG&E Calculations. $5,301 | Source: PG&E Calculations. $7,668 |
| Measure Incremental Cost ($/unit): | Source: PG&E Calculations. $233 | Source: PG&E Calculations. $550 | Source: PG&E Calculations. $866 |
| Effective Useful Life (years): | Source: DEER. EUL for Retrofit EUL for Replace on Burnout (ROB) = 12 years, and New Construction (NEW) = 12 years. | Source: DEER. EUL for Retrofit EUL for Replace on Burnout (ROB) = 12 years, and New Construction (NEW) = 12 years. | Source: DEER. EUL for Retrofit EUL for Replace on Burnout (ROB) = 12 years, and New Construction (NEW) = 12 years. |
| Program Type: | Retrofit Replace on Burnout (ROB), and New Construction (NEW). | Retrofit Replace on Burnout (ROB), and New Construction (NEW). | Retrofit Replace on Burnout (ROB), and New Construction (NEW). |
| Net-to-Gross Ratios: | Source: 2011 DEER, Com-Default>2yrs, 0.60 | Source: 2011 DEER, Com-Default>2yrs, 0.60 | Source: 2011 DEER, Com-Default>2yrs, 0.60 |
| Important Comments: |  |  |  |

# At a Glance Summary: CEE Tier III Ice Machines

| Applicable Measure Codes: | FS-D15 | FS-D16 | FS-D17 | FS-D18 |
| --- | --- | --- | --- | --- |
| Measure Description: | Commercial Ice Machine, 101 – 200 lbs/day | Commercial Ice Machine, 201 – 300 lbs/day | Commercial Ice Machine, 301 – 400 lbs/day | Commercial Ice Machine, 401 – 500 lbs/day |
| Energy Impact Common Units: | Ice Machine | Ice Machine | Ice Machine | Ice Machine |
| Base Case Description: | Source: PG&E Calculations. Commercial Ice Machine, 101 – 200 lbs/day | Source: PG&E Calculations. Commercial Ice Machine, 201 – 300 lbs/day | Source: PG&E Calculations. Commercial Ice Machine, 301 – 400 lbs/day | Source: PG&E Calculations. Commercial Ice Machine, 401 – 500 lbs/day |
| Base Case Energy Consumption: | Source: PG&E Calculations. 4,503 kWh/yr | Source: PG&E Calculations. 5,550 kWh/yr | Source: PG&E Calculations. 6,946 kWh/yr | Source: PG&E Calculations. 7,878 kWh/yr |
| Measure Energy Consumption: | Source: PG&E Calculations. 3,825 kWh/yr | Source: PG&E Calculations. 4,719 kWh/yr | Source: PG&E Calculations. 5,907 kWh/yr | Source: PG&E Calculations. 6,720 kWh/yr |
| Energy Savings (Base Case – Measure) | Source: PG&E Calculations. 678 kWh/yr | Source: PG&E Calculations. 832 kWh/yr | Source: PG&E Calculations. 1,040 kWh/yr | Source: PG&E Calculations. 1,158 kWh/yr |
| Costs Common Units: | Ice Machine | Ice Machine | Ice Machine | Ice Machine |
| Base Case Equipment Cost ($/unit): | Source: PG&E Calculations. $1,976 | Source: PG&E Calculations. $2,442 | Source: PG&E Calculations. $2,459 | Source: PG&E Calculations. $2,583 |
| Measure Equipment Cost ($/unit): | Source: PG&E Calculations. $2,106 | Source: PG&E Calculations. $3,125 | Source: PG&E Calculations. $3,711 | Source: PG&E Calculations. $4,057 |
| Measure Incremental Cost ($/unit): | Source: PG&E Calculations. $130 | Source: PG&E Calculations. $683 | Source: PG&E Calculations. $1,252 | Source: PG&E Calculations. $1,474 |
| Effective Useful Life (years): | Source: DEER. EUL for Retrofit EUL for Replace on Burnout (ROB) = 12 years, and New Construction (NEW) = 12 years. | Source: DEER. EUL for Retrofit EUL for Replace on Burnout (ROB) = 12 years, and New Construction (NEW) = 12 years. | Source: DEER. EUL for Retrofit EUL for Replace on Burnout (ROB) = 12 years, and New Construction (NEW) = 12 years. | Source: DEER. EUL for Retrofit EUL for Replace on Burnout (ROB) = 12 years, and New Construction (NEW) = 12 years. |
| Program Type: | Retrofit Replace on Burnout (ROB), and New Construction (NEW). | Retrofit Replace on Burnout (ROB), and New Construction (NEW). | Retrofit Replace on Burnout (ROB), and New Construction (NEW). | Retrofit Replace on Burnout (ROB), and New Construction (NEW). |
| Net-to-Gross Ratios: | Source: 2011 DEER, Com-Default>2yrs, 0.60 | Source: 2011 DEER, Com-Default>2yrs, 0.60 | Source: 2011 DEER, Com-Default>2yrs, 0.60 | Source: 2011 DEER, Com-Default>2yrs, 0.60 |
| Important Comments: |  |  |  |  |

# At a Glance Summary: CEE Tier III Ice Machines (Cont)

| Applicable Measure Codes: | FS-D19 | FS-D20 | FS-D21 |
| --- | --- | --- | --- |
| Measure Description: | Commercial Ice Machine, 501 – 1,000 lbs/day | Commercial Ice Machine, 1,001 – 1,500 lbs/day | Commercial Ice Machine, > 1,500 lbs/day |
| Energy Impact Common Units: | Ice Machine | Ice Machine | Ice Machine |
| Base Case Description: | Source: PG&E Calculations. Commercial Ice Machine, 501 – 1,000 lbs/day | Source: PG&E Calculations. Commercial Ice Machine, 1,001 – 1,500 lbs/day | Source: PG&E Calculations. Commercial Ice Machine, > 1,500 lbs/day |
| Base Case Energy Consumption: | Source: PG&E Calculations. 12,452 kWh/yr | Source: PG&E Calculations. 17,452 kWh/yr | Source: PG&E Calculations. 24,432 kWh/yr |
| Measure Energy Consumption: | Source: PG&E Calculations. 10,645 kWh/yr | Source: PG&E Calculations. 14,851 kWh/yr | Source: PG&E Calculations. 20,791 kWh/yr |
| Energy Savings (Base Case – Measure) | Source: PG&E Calculations. 1,807 kWh/yr | Source: PG&E Calculations. 2,601 kWh/yr | Source: PG&E Calculations. 3,641 kWh/yr |
| Costs Common Units: | Ice Machine | Ice Machine | Ice Machine |
| Base Case Equipment Cost ($/unit): | Source: PG&E Calculations. $3,494 | Source: PG&E Calculations. $4,751 | Source: PG&E Calculations. $6,802 |
| Measure Equipment Cost ($/unit): | Source: PG&E Calculations. $4,081 | Source: PG&E Calculations. $5,591 | Source: PG&E Calculations. $8,135 |
| Measure Incremental Cost ($/unit): | Source: PG&E Calculations. $586 | Source: PG&E Calculations. $839 | Source: PG&E Calculations. $1,333 |
| Effective Useful Life (years): | Source: DEER. EUL for Retrofit EUL for Replace on Burnout (ROB) = 12 years, and New Construction (NEW) = 12 years. | Source: DEER. EUL for Retrofit EUL for Replace on Burnout (ROB) = 12 years, and New Construction (NEW) = 12 years. | Source: DEER. EUL for Retrofit EUL for Replace on Burnout (ROB) = 12 years, and New Construction (NEW) = 12 years. |
| Program Type: | Retrofit Replace on Burnout (ROB), and New Construction (NEW). | Retrofit Replace on Burnout (ROB), and New Construction (NEW). | Retrofit Replace on Burnout (ROB), and New Construction (NEW). |
| Net-to-Gross Ratios: | Source: 2011 DEER, Com-Default>2yrs, 0.60 | Source: 2011 DEER, Com-Default>2yrs, 0.60 | Source: 2011 DEER, Com-Default>2yrs, 0.60 |
| Important Comments: |  |  |  |

# At-A-Glance Measure List: Energy Star® Ice Machines

| Measure Code | Measure Name | Run ID | Measure Description | Target Sector Code | Target Sector | Climate Zone (CZ) | Peak Electric Demand Reduction (kW/unit) | Electric Savings (kWh/unit) | Gas Savings (therms/unit) | Base Case Cost ($/unit) | Measure Cost ($/unit) | Measure Incremental Cost ($/unit) | Effective Useful Life (years) | Program Type (NEW, ROB, RET) | NTG | Unit Definition | Total Number of Units | Delivery Methods |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FS-D8 | Commercial Ice Machine, | 8 | 101 - 200 lbs/day | ALL | All | ALL | 0.038 | 331 | 0 | 1,976 | 2,165 | 189 | 12 years | Ret | 0.6 | Per Ice Machine | 1 | Downstream Commercial |
| FS-D9 | Commercial Ice Machine, | 9 | 201 - 300 lbs/day | ALL | All | ALL | 0.063 | 557 | 0 | 2,442 | 3,260 | 818 | 12 years | Ret | 0.6 | Per Ice Machine | 1 | Downstream Commercial |
| FS-D10 | Commercial Ice Machine, | 4 | 301 - 400 lbs/day | ALL | All | ALL | 0.07 | 685 | 0 | 2,459 | 2,740 | 281 | 12 years | Ret | 0.6 | Per Ice Machine | 1 | Downstream Commercial |
| FS-D11 | Commercial Ice Machine, | 5 | 401 - 500 lbs/day | ALL | All | ALL | 0.091 | 795 | 0 | 2,583 | 2,646 | 63 | 12 years | Ret | 0.6 | Per Ice Machine | 1 | Downstream Commercial |
| FS-D12 | Commercial Ice Machine, | 7 | 501 – 1,000 lbs/day | ALL | All | ALL | 0.14 | 1,263 | 0 | 3,494 | 3,728 | 233 | 12 years | Ret | 0.6 | Per Ice Machine | 1 | Downstream Commercial |
| FS-D13 | Commercial Ice Machine, | 3 | 1,001 – 1,500 lbs/day | ALL | All | ALL | 0.18 | 1,574 | 0 | 4,751 | 5,301 | 550 | 12 years | Ret | 0.6 | Per Ice Machine | 1 | Downstream Commercial |
| FS-D14 | Commercial Ice Machine, | 13 | > 1,500 lbs/day | ALL | All | ALL | 0.252 | 2,204 | 0 | 6,802 | 7,668 | 866 | 12 years | Ret | 0.6 | Per Ice Machine | 1 | Downstream Commercial |

# Work Paper Approvals

| Charles Harmstead Supervisor, Energy Efficiency | Date |
| --- | --- |

# Document Revision History

Revision # 		Date 		   Description		   Author (Company)

| Revision 0 | 10/13/2010 | #### Converted to SDGE work paper from PGE work paper. PGE COFST108 Revision 1 dated January 1, 2008 | Charles Harmstead, SDGE |
| --- | --- | --- | --- |
| Revision 1 | 10/15/2010 | NTG was updated to reflect DEER 2008 Saving was corrected | Lucie Sidibe, SDGE |
| Revision 2 | 26 June 2012 | NTG was updated to reflect DEER 2011 (removed reference sentence from pg. v, August 28,2012) | Peter Ford (SDGE) |

# Table of Contents

At a Glance Summary: Energy Star® Ice Machines	i

At a Glance Summary: Energy Star® Ice Machines (Cont)	ii

At a Glance Summary: CEE Tier III Ice Machines	iii

At a Glance Summary: CEE Tier III Ice Machines (Cont)	iv

At-A-Glance Measure List: Energy Star® Ice Machines	v

Work Paper Approvals	vi

Document Revision History	vi

Table of Contents	vii

List of Tables	viii

Section 1. General Measure & Baseline Data	1

1.1 Measure Description & Background	1

1.2 DEER Differences Analysis	3

1.3 Codes & Standards Requirements Analysis	3

1.4 EM&V, Market Potential, and Other Studies	4

1.5 Base Cases for Savings Estimates: Existing & Above Code	4

1.6 Base Cases & Measure Effective Useful Lives	4

1.7 Net-to-Gross Ratios for Different Program Strategies	5

Section 2. Calculation Methods	5

2.1 Electric Energy Savings Estimation Methodologies	5

2.2. Demand Reduction Estimation Methodologies	7

2.3. Gas Energy Savings Estimation Methodologies	8

Section 3. Load Shapes	8

3.1 Base Cases Load Shapes	9

3.2 Measure Load Shapes	9

Section 4. Base Case & Measure Costs	9

4.1 Base Cases Costs	9

4.2 Measure Costs	10

4.3 Incremental & Full Measure Costs	10

References	12

Appendix A	13

# List of Tables

Table 1. Energy Star® Energy Efficiency Requirements for Commercial Ice Machines.	2

Table 2. CEE Tier III Energy Efficiency Requirements for Commercial Ice Machines.	2

Table 3. Title 20 Energy Efficiency Requirements for Commercial Ice Machines.	4

Table 4. Energy Star® Commercial Air-Cooled Ice Machine Cost Effectiveness Example	6

Table 5. CEE Tier III Commercial Air-Cooled Ice Machine Cost Effectiveness Example	7

Table 6. Energy Star® Commercial Air-Cooled Ice Machine Demand Savings	8

Table 7. CEE Tier III Commercial Air-Cooled Ice Machine Demand Savings	8

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper documents the rationale for the 2008 Commercial Ice Machine measures as listed in the Commercial Food Service Catalog. The Commercial Food Service Catalog is part of San Diego Gas and Electric Company’s Customer Energy Efficiency Program. SDGE offers incentives to non-residential customers for installing qualifying lighting, refrigeration, air-conditioning, food service, and agricultural equipment.

Catalog Description –

New Specifications and Rebate Levels are Effective for all Purchases and Installations Effective 1/1/2008. This specification covers machines generating 60 grams (2 oz.) or lighter ice cubes, as well as flaked, crushed and fragmented ice makers. Performance data is based on ARI Standard 810. Only air-cooled machines (self contained, icemaker heads, or remote condensing) are eligible. The efficiency specifications must be equivalent to ENERGY STAR® or CEE Tier III. Visit www.ari.org for product information and testing procedures. Specifications are available at www.energyatar.gov/cfs and www.cee1.org. To qualify, the entire ARI tested Ice Making system must be purchased to qualify. Remote machines must be purchased with qualifying remote condenser or remote condenser/compressor unit.

Program Restrictions and Guidelines

Terms and Conditions

This incentive applies towards the purchase of new or replacement energy efficient Air-cooled ice machines. Used or rebuilt equipment is not eligible. Customers must provide proof that the appliance meets the energy efficiency specifications listed in Tables 1 and 2.

This specification covers machines generating 60 grams (2 oz.) or lighter ice cubes, as well as flaked, crushed, or fragmented ice machines that meet the Energy Star® energy efficiency thresholds by Ice harvest (IHR) rate1 listed in Table 1, or the Consortium for Energy Efficiency (CEE) Tier III energy efficiency thresholds by Ice harvest (IHR) rate2 listed in Table 2. Only air cooled machines (icemaker heads, self-contained unites, and remote condensing units) are eligible for incentives. Performance data is based on ARI Standard 810. 3

Table 1. Energy Star® Energy Efficiency Requirements for Commercial Ice Machines.

| Equipment Type | Ice Harvest Rate (lbs ice/day) | Energy Use Limit (kWh/100 lbs ice) | Potable Water Use Limit (gal/100 lbs ice) |
| --- | --- | --- | --- |
| Ice Maker Head (IMH) | < 450 | 9.23 – 0.0077 × H a | ≤ 25 |
|  | ≥ 450 | 6.20 – 0.0010 × H a | ≤ 25 |
| Remote Condensing Unit (RCU) without remote compressor | < 1,000 | 8.05 – 0.0035 × H a | ≤ 25 |
|  | ≥ 1,000 | 4.64 | ≤ 25 |
| Remote Condensing Unit (RCU) with remote compressor | < 934 | 8.05 – 0.0035 × H a | ≤ 25 |
|  | ≥ 934 | 4.82 | ≤ 25 |
| Self Contained Unit (SCU) | < 175 | 16.7 – 0.0436 × H a | ≤ 35 |
|  | ≥ 175 | 9.11 | ≤ 35 |

a H = Ice Harvest Rate (IHR) for the commercial ice machine as determined by applying ARI Standard 810.

Table 2. CEE Tier III Energy Efficiency Requirements for Commercial Ice Machines.

| Equipment Type | Ice Harvest Rate (lbs ice/day) | Energy Use Limit (kWh/100 lbs ice) | Potable Water Use Limit (gal/100 lbs ice) |
| --- | --- | --- | --- |
| Ice Maker Head (IMH) | < 450 | 8.72 – 0.0073 × H a | ≤ 20 |
|  | ≥ 450 | 5.86 – 0.0009 × H a | ≤ 20 |
| Remote Condensing Unit (RCU) without remote compressor | < 1,000 | 7.52 – 0.0032 × H a | ≤ 20 |
|  | ≥ 1,000 | 4.34 | ≤ 20 |
| Remote Condensing Unit (RCU) with remote compressor | < 934 | 7.52 – 0.0032 × H a | ≤ 20 |
|  | ≥ 934 | 4.51 | ≤ 20 |
| Self Contained Unit (SCU) | < 175 | 15.3 – 0.0399 × H a | ≤ 30 |
|  | ≥ 175 | 8.33 | ≤ 30 |

a H = Ice Harvest Rate (IHR) for the commercial ice machine as determined by applying ARI Standard 810.

Market Applicability

Hospitals account for 39.4 percent of all commercial icemaker purchases, followed by hotels (22.3 percent), restaurants (13.8 percent), retail outlets (8.5 percent), schools (8.5 percent), offices (4.3 percent), and grocery stores (3.2 percent).4

Technical Description

Commercial icemakers represent approximately 11 percent of all commercial refrigeration energy use. The average annual energy use of a 500 lb/day air-cooled icemaker is 5,000 kWh with a potential increase in efficiency of 15 percent (less than a two-year payback). 4 Approximately 64 percent of the commercial ice machine market is self-contained cube making units; the rest are ice making heads and remote condensing units. The PG&E Food Service Technology Center (FSTC) estimates that there are more than 200,000 ice machines in California, with up to 100,000 in PG&E service territory.

Icemakers use a substantial amount of energy in order to freeze water and maintain the ice as separate cubes. Reductions are possible with the use of high efficiency motors in condenser fans and compressors, thicker insulation and reduced evaporator thermal cycling. The energy use in a commercial icemaker varies from product to product, depending on the condenser and the type of ice produced. 5

The Federal Energy Management Program launched a specification for energy efficient commercial ice cube machines in 2000. 6 The specification included different energy consumption thresholds based on the size (lbs/day) and type (icemaker head, self-contained, or remote condensing unit) of ice machine. The Consortium for Energy Efficiency (CEE) launched a similar specification for commercial ice makers in 2002. CEE's specification has three Tiers of energy efficiency, Tier I was based on the top 25 percent of products in the market at the time of the specification launch in 2003, which is roughly equal to the Federal Management Energy Program recommendation. Tier II is approximately 10 percent more efficient than Tier I and Tier III is approximately 10 percent more efficient than Tier II. 4

Energy Star® has recently developed a new specification for commercial air-cooled ice machines, based on the CEE Tier II specification. The Energy Star® specification will officially launch on January 1, 2008.1 After January 1, 2008, the California Energy Commission (CEC) will require ice machines to meet the CEE Tier I specifications to comply with Title 20 changes.7 With the new baseline per Title 20, the commercial food service program will move to promote the higher efficiency levels represented by Energy Star® and CEE Tier III.

## 1.2 DEER Differences Analysis

The DEER database does not contain an energy-efficient commercial ice machine measure.

## 1.3 Codes & Standards Requirements Analysis

Title 20: Under this regulation, all commercial ice machines manufactured after January 1, 2008 are required to meet the specifications outlined in Table 3. 7

Table 3. Title 20 Energy Efficiency Requirements for Commercial Ice Machines.

| Equipment Type | Ice Harvest Rate (lbs ice/day) | Energy Use Limit (kWh/100 lbs ice) |
| --- | --- | --- |
| Ice Maker Head (IMH) | < 450 | 10.26 – 0.0086 × H a |
|  | ≥ 450 | 6.89 – 0.0011 × H a |
| Remote Condensing Unit (RCU) without remote compressor | < 1,000 | 8.85 – 0.0038 × H a |
|  | ≥ 1,000 | 5.10 |
| Remote Condensing Unit (RCU) with remote compressor | < 934 | 8.85 – 0.0038 × H a |
|  | ≥ 934 | 5.30 |
| Self Contained Unit (SCU) | < 175 | 18.0 – 0.0469 × H a |
|  | ≥ 175 | 9.80 |

a H = Ice Harvest Rate (IHR) for the commercial ice machine as determined by applying ARI Standard 810.

Title 24: These measures do not fall under Title 24 of the California Energy Regulations.

Federal Standards: These measures do not currently fall under Federal DOE or EPA Energy Regulations.

ARI Standard 810, Performance Rating of Automatic Commercial Ice-Makers, is considered the industry standard for estimating commercial ice machine energy use. 3 The ARI test data was used to estimate the energy consumption of the base case and measure equipment.

## 1.4 EM&V, Market Potential, and Other Studies

There were no specific EM&V studies identified that addressed cooking measures in the commercial sector.

## 1.5 Base Cases for Savings Estimates: Existing & Above Code

The base case for both existing units and above code savings was the CEE Tier I specifications for commercial ice machines. 2 This is consistent with the California Energy Commission (CEC) Title 20 regulations for commercial ice machines. 7

## 1.6 Base Cases & Measure Effective Useful Lives

The DEER Measure Cost Data Users Guide, version 2.01, defines the following terms:

- Retrofit (RET) – replacing a working technology prior to failure.

- Replace on Burnout (ROB) – replacing a technology at the end of its useful life.

- New Construction (NEW) – installing a technology in a new construction or major renovation project.

The 2003 Energy Efficiency Policy Manual Table 4.1 and 2005 DEER database record, D03-907, show an EUL of 12 years for all commercial food service appliance measures, including ice machines. 8 Commercial ice machines are classed as suitable for RET, ROB, and NEW installations.

## 1.7 Net-to-Gross Ratios for Different Program Strategies

The Net-to-Gross ratio for this measure is taken from the 2011 DEER NTG Table as 0.60 using the default value for measures that have been in the program over two years.

# Section 2. Calculation Methods

## 2.1 Electric Energy Savings Estimation Methodologies

In an analysis of the range of products currently on the market, American Council for an Energy Efficient-Economy (ACEEE) research indicates that the highest performing models on the market are 18.46 percent more efficient than the worst performing models, with a payback period of 1.1 years or less.9 Manufacturers have also indicated that they have the technical capability to exceed current efficiency levels through the use of high efficiency compressors and fan motors, thicker insulation, and other measures.

The industry standard for energy use and performance of commercial ice machines is ARI Standard 810. Table 4 lists the cost effectiveness of Energy Star® ice machines, based on the individual machines operating at 75% of their rated maximum capacity. Table 5 lists the cost effectiveness of CEE Tier III ice machines, based on the individual machines operating at 75% of their rated maximum capacity.

Table 4. Energy Star® Commercial Air-Cooled Ice Machine Cost Effectiveness Example

| Performance | FS-D8 | FS-D9 | FS-D10 | FS-D11 | FS-D12 | FS-D13 | FS-D14 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ice Harvest Rate (IHR) (lbs per 24 hrs.) | 101-200 | 201-300 | 301-400 | 401-500 | 501-1,000 | 1,001-1,500 | > 1,500 |
| Average IHR Used in Energy Calculations (lbs/day) | 150 | 250 | 350 | 450 | 750 | 1,250 | 1,750 |
| Baseline Model Energy Usage (kWh/100 lbs) | 10.97 | 8.11 | 7.25 | 6.40 | 6.07 | 5.10 | 5.10 |
| Energy Efficient Model Energy Usage (kWh/100 lbs) | 10.16 | 7.31 | 6.54 | 5.75 | 5.45 | 4.64 | 4.64 |
| Baseline Model Daily Energy Consumption (kWh) | 12.3 | 15.2 | 19.0 | 21.6 | 34.1 | 47.8 | 66.9 |
| Energy Efficient Model Daily Energy Consumption (kWh) | 11.4 | 13.7 | 17.2 | 19.4 | 30.7 | 43.5 | 60.9 |
| Baseline Model Average Demand (kW) | 0.514 | 0.634 | 0.793 | 0.899 | 1.421 | 1.992 | 2.789 |
| Energy Efficient Model Average Demand (kW) | 0.476 | 0.571 | 0.715 | 0.809 | 1.277 | 1.813 | 2.538 |
| Estimated Demand Reduction (kW) | 0.038 | 0.063 | 0.078 | 0.091 | 0.144 | 0.180 | 0.252 |
| Baseline Model Annual Energy Consumption (kWh/yr) | 4,503 | 5,550 | 6,946 | 7,878 | 12,452 | 17,452 | 24,432 |
| Energy Efficient Model Annual Energy Consumption (kWh/yr) | 4,172 | 4,999 | 6,261 | 7,083 | 11,190 | 15,878 | 22,229 |
| Estimated Annual Energy Savings (kWh/yr) | 331 | 551 | 685 | 795 | 1,263 | 1,574 | 2,204 |
| Electric Cost ($/kWh) | $0.13 | $0.13 | $0.13 | $0.13 | $0.13 | $0.13 | $0.13 |
| Baseline Model Annual Energy Cost ($/yr) | $585 | $722 | $903 | $1,024 | $1,619 | $2,269 | $3,176 |
| Energy Efficient Model Annual Energy Cost ($/yr) | $542 | $650 | $814 | $921 | $1,455 | $2,064 | $2,890 |
| Estimated Annual Energy Cost Savings ($/yr) | $43 | $72 | $89 | $103 | $164 | $205 | $286 |
| Estimated Incremental Cost b | SEE APPENDIX A |  |  |  |  |  |  |
| Estimated Useful Life (EUL) c | 12 | 12 | 12 | 12 | 12 | 12 | 12 |

a Incremental measure cost was determined through comparison of an average of published pricing listed in APPENDIX A.

b The estimated useful life is based the 2003 Energy Efficiency Policy Manual Table 4.1 and is consistent with DEER EUL estimates.

Table 5. CEE Tier III Commercial Air-Cooled Ice Machine Cost Effectiveness Example

| Performance | FS-D15 | FS-D16 | FS-D17 | FS-D18 | FS-D19 | FS-D20 | FS-D21 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ice Harvest Rate (IHR) (lbs per 24 hrs.) | 101-200 | 201-300 | 301-400 | 401-500 | 501-1,000 | 1,001-1,500 | > 1,500 |
| Average IHR Used in Energy Calculations (lbs/day) | 150 | 250 | 350 | 450 | 750 | 1,250 | 1,750 |
| Baseline Model Energy Usage (kWh/100 lbs) | 10.97 | 8.11 | 7.25 | 6.40 | 6.07 | 5.10 | 5.10 |
| Energy Efficient Model Energy Usage (kWh/100 lbs) | 9.32 | 6.90 | 6.17 | 5.46 | 5.19 | 4.34 | 4.34 |
| Baseline Model Daily Energy Consumption (kWh) | 12.3 | 15.2 | 19.0 | 21.6 | 34.1 | 47.8 | 66.9 |
| Energy Efficient Model Daily Energy Consumption (kWh) | 10.5 | 12.9 | 16.2 | 18.4 | 29.2 | 40.7 | 57.0 |
| Baseline Model Average Demand (kW) | 0.514 | 0.634 | 0.793 | 0.899 | 1.421 | 1.992 | 2.789 |
| Energy Efficient Model Average Demand (kW) | 0.437 | 0.539 | 0.674 | 0.767 | 1.215 | 1.695 | 2.373 |
| Estimated Demand Reduction (kW) | 0.077 | 0.095 | 0.119 | 0.132 | 0.206 | 0.297 | 0.416 |
| Baseline Model Annual Energy Consumption (kWh/yr) | 4,503 | 5,550 | 6,946 | 7,878 | 12,452 | 17,452 | 24,432 |
| Energy Efficient Model Annual Energy Consumption (kWh/yr) | 3,825 | 4,719 | 5,907 | 6,720 | 10,645 | 14,851 | 20,791 |
| Estimated Annual Energy Savings (kWh/yr) | 678 | 832 | 1,040 | 1,158 | 1,807 | 2,601 | 3,641 |
| Electric Cost ($/kWh) | $0.13 | $0.13 | $0.13 | $0.13 | $0.13 | $0.13 | $0.13 |
| Baseline Model Annual Energy Cost ($/yr) | $585 | $722 | $903 | $1,024 | $1,619 | $2,269 | $3,176 |
| Energy Efficient Model Annual Energy Cost ($/yr) | $497 | $613 | $768 | $874 | $1,384 | $1,931 | $2,703 |
| Estimated Annual Energy Cost Savings ($/yr) | $88 | $108 | $135 | $151 | $235 | $338 | $473 |
| Estimated Incremental Cost b | SEE APPENDIX A |  |  |  |  |  |  |
| Estimated Useful Life (EUL) c | 12 | 12 | 12 | 12 | 12 | 12 | 12 |

a Incremental measure cost was determined through comparison of an average of published pricing listed in APPENDIX A.

b The estimated useful life is based the 2003 Energy Efficiency Policy Manual Table 4.1 and is consistent with DEER EUL estimates.

## 2.2. Demand Reduction Estimation Methodologies

The demand reduction estimation is based on ARI reported data for standard efficiency ice machines and for high-efficiency ice machines. The measured data are derived from tests conducted under ARI Standard 810.3 The estimated demand reduction was based on the average demand for baseline and energy efficient model ice machines.

ARI Standard 810 provides standard conditions under which ice machine energy use is measured. The estimated demand reduction of 100 Watts for a 450-pound ice machine was based on data from tests of standard efficiency and high-efficiency ice machines. Applying a Coincidence Factor of 0.9 per the DEER methodology 8 yields a Demand Savings of 82 Watts for the 450-pound Energy Star® ice machine. Table 6 summarizes the Demand Savings for the various sizes of Energy Star® ice machines. Table 7 summarizes the Demand Savings for the various sizes of CEE Tier III ice machines.

Table 6. Energy Star® Commercial Air-Cooled Ice Machine Demand Savings

| Performance(IHR) | FS-D8 | FS-D9 | FS-D10 | IHR | IHR | IHR | IHR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ice Harvest Rate (IHR) (lbs per 24 hrs.) | 101-200 | 201-300 | 301-400 | 401-500 | 501-1,000 | 1,001-1,500 | > 1,500 |
| Average IHR Used in Energy Calculations (lbs/day) | 150 | 250 | 350 | 450 | 750 | 1,250 | 1,750 |
| Baseline Model Average Demand (kW) | 0.514 | 0.634 | 0.793 | 0.899 | 1.421 | 1.992 | 2.789 |
| Energy Efficient Model Average Demand (kW) | 0.476 | 0.571 | 0.715 | 0.809 | 1.277 | 1.813 | 2.538 |
| Estimated Demand Reduction (kW) | 0.038 | 0.063 | 0.078 | 0.091 | 0.144 | 0.180 | 0.252 |
| Demand Savings (kW) a | 0.034 | 0.057 | 0.070 | 0.082 | 0.130 | 0.162 | 0.226 |

a Demand Savings includes a 0.9 coincidence factor per DEER methodology.

Table 7. CEE Tier III Commercial Air-Cooled Ice Machine Demand Savings

| Performance | IHR | IHR | IHR | IHR | IHR | IHR | IHR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ice Harvest Rate (IHR) (lbs per 24 hrs.) | 101-200 | 201-300 | 301-400 | 401-500 | 501-1,000 | 1,001-1,500 | > 1,500 |
| Average IHR Used in Energy Calculations (lbs/day) | 150 | 250 | 350 | 450 | 750 | 1,250 | 1,750 |
| Baseline Model Average Demand (kW) | 0.514 | 0.634 | 0.793 | 0.899 | 1.421 | 1.992 | 2.789 |
| Energy Efficient Model Average Demand (kW) | 0.437 | 0.539 | 0.674 | 0.767 | 1.215 | 1.695 | 2.373 |
| Estimated Demand Reduction (kW) | 0.077 | 0.095 | 0.119 | 0.132 | 0.206 | 0.297 | 0.416 |
| Demand Savings (kW) a | 0.070 | 0.085 | 0.107 | 0.119 | 0.186 | 0.267 | 0.374 |

a Demand Savings includes a 0.9 coincidence factor per DEER methodology.

## 2.3. Gas Energy Savings Estimation Methodologies

There were no gas energy savings associated with these measures.

# Section 3. Load Shapes

Load Shapes are an important part of the life-cycle cost analysis of any energy efficiency program portfolio.  The net benefits associated with a measure are based on the amount of energy saved and the avoided cost per unit of energy saved.  For electricity, the avoided cost varies hourly over an entire year.  Thus, the net benefits calculation for a measure requires both the total annual energy savings (kWh) of the measure and the distribution of that savings over the year.  The distribution of savings over the year is represented by the measure’s load shape.  The measure’s load shape indicates what fraction of annual energy savings occurs in each time period of the year.  An hourly load shape indicates what fraction of annual savings occurs for each hour of the year.  A Time-of-Use (TOU) load shape indicates what fraction occurs within five or six broad time-of-use periods, typically defined by a specific utility rate tariff.  Formally, a load shape is a set of fractions summing to unity, one fraction for each hour or for each TOU period.  Multiplying the measure load shape with the hourly avoided cost stream determines the average avoided cost per kWh for use in the life cycle cost analysis that determines a measure’s Total Resource Cost (TRC) benefit.

## 3.1 Base Cases Load Shapes

The base case load shape would be expected to follow a typical nonresidential foodservice end use load shape.

Commercial ice machine load shapes  differ among food service facilities (quick service, casual dining, hotels, college, schools, hospitals etc) depending on daily hours of operation, serving periods, day-of-week, and facility location (city downtown, suburban mall, access to interstate highways, etc.). Consequently, applicable average TOU and hourly load shapes for ice machines are unavailable. Commercial ice machines will generally consume more energy during operating hours, when the demand for ice is the greatest.

## 3.2 Measure Load Shapes

For purposes of the net benefits estimates in the E3 calculator, what is required is the load shape that ideally represents the difference between the base equipment and the installed energy efficiency measure.  This difference load profile is what is called the Measure Load Shape and would be the preferred load shape for use in the net benefits calculations.

The measure load shape for this measure is determined by the E3 calculator based on the applicable nonresidential market sector and the foodservice end-use.

The electric demand profile for the high-efficiency ice machines is expected to be the same as the Base Case. The profile will vary as explained in Section 3.1. The Measure Load Shape for the high-efficiency ice machine will use less energy and have a slightly lower demand profile.

# Section 4. Base Case & Measure Costs

Star® and CEE Tier III ice machines typically have a higher list price than standard efficiency ice machines. Models that meet this requirement incorporate better insulation, reducing heat loss, and more efficient components such as ECM evaporator fan motors and high-efficiency compressors.

## 4.1 Base Cases Costs

The Base Case costs include only the equipment. Energy Star® and CEE Tier III ice machines require no additional labor or maintenance compared to base case ice machines. Since this measure is applicable for RET, ROB and NEW installations, the installation and maintenance costs are expected to be the same for the customer. The estimated equipment cost is based on recent list cost data for ice machines and applying an industry-standard 40% discount to the manufacturer published list prices. 10-13

## 4.2 Measure Costs

The Measure costs include only the equipment, as explained in Section 4.1. The estimated equipment cost is based on recent list cost data and applying an industry-standard 40% discount to the manufacturer published list prices (see Appendix A). 10-13

## 4.3 Incremental & Full Measure Costs

Full measure costs are used in the analysis.

Index

DEER Measure Cost Data Users Guide, 4

Food Service Equipment, ii, iii, iv, v

load shape, 8

Measure Load Shape, 9

New Construction, ii, iii, iv, v, 4

Replace on Burnout, ii, iii, iv, v, 4

Retrofit, ii, iii, iv, v, 4

# References

- EPA 2008. Energy Star® Program Requirements for Commercial Ice Machines.

- Consortium for Energy Efficiency (CEE), 2007. Commercial Ice-Cube Machine Specifications.

- Air-Conditioning & Refrigeration Institute (ARI), 2003. Standard 810 Performance Rating of Automatic Commercial Ice-Makers.

- Consortium for Energy Efficiency (CEE) Commercial Ice Machine Program FAQ.

- ADL 1996. Energy Savings Potential for Commercial Refrigeration Equipment, pp. 39-49.

- Federal Energy Management Program (FEMP) Purchasing Specifications for Commercial Ice-Cube Machines. http://www1.eere.energy.gov/femp/procurement/eep_ice_makers.html

- 2007 California Energy Commission (CEC) Title 20 Appliance Efficiency Regulations, CEC 400-2007-016, p. 112.

- 2004-2005 Database for Energy Efficiency Resources (DEER) Update Study Final Report, pp. 3-15 to 3-18.

- American Council for an Energy-Efficient Economy (ACEEE), 2002. Packaged Commercial Refrigeration Equipment: A Briefing Report For Program Planners And Implementers.

- Hoshizaki, 2007. 2007 Equipment Price List.

- Ice-O-Matic 2007. Domestic Suggested List Price Schedule Commercial Ice Machines and Accessories.

- Manitowoc, 2008. Suggested List Price Standard Equipment.

- Scotsman, 2007. Suggested List Price Schedule Commercial Products.

# Appendix A

Equipment Cost Data for Energy Star® Ice Machines

| Make Energy Efficient | Model | Ice Harvest Rate (lb/24h) | Cost($)* |
| --- | --- | --- | --- |
| Manitowoc | QY-214A | 155 | $3,608 |
| Hoshizaki | DCM-270BAH-OS | 220 | $8,890 |
| Manitowoc | SD-0322A | 250 | $3,793 |
| Hoshizaki | KM-320MAH | 278 | $3,620 |
| Hoshizaki | F-450MAH | 355 | $5,040 |
| Manitowoc | SY-0454A | 365 | $4,094 |
| Scotsman | CO530MR-1A/ERC111-1A | 460 | $4,410 |
| Scotsman | CO630MR-32A/ERC111-1A | 616 | $5,129 |
| Hoshizaki | KMS-750MLH | 700 | $5,900 |
| Scotsman | CO1030MR-32A/ERC311 | 851 | $7,610 |
| Hoshizaki | KM1340MRH/URC-14F | 1251 | $7,620 |
| Hoshizaki | KM-1601MRH3/URC-21F | 1459 | $10,050 |
| Hoshizaki | KM-2000SRH3/URC-20F | 1748 | $12,780 |
| Average List Price of 101-200 lb Energy Efficient Ice Machine |  |  | $3,608 |
| Average Estimated Purchase Price of 101-200 lb Energy Efficient Ice Machine** |  |  | $2,165 |
| Average List Price of 201-300 lb Energy Efficient Ice Machine |  |  | $5,434 |
| Average Estimated Purchase Price of 201-300 lb Energy Efficient Ice Machine** |  |  | $3,260 |
| Average List Price of 301-400 lb Energy Efficient Ice Machine |  |  | $4,567 |
| Average Estimated Purchase Price of 301-400 lb Energy Efficient Ice Machine** |  |  | $2,740 |
| Average List Price of 401-500 lb Energy Efficient Ice Machine |  |  | $4,410 |
| Average Estimated Purchase Price of 401-500 lb Energy Efficient Ice Machine** |  |  | $2,646 |
| Average List Price of 501-1,000 lb Energy Efficient Ice Machine |  |  | $6,213 |
| Average Estimated Purchase Price of 501-1,000 lb Energy Efficient Ice Machine** |  |  | $3,728 |
| Average List Price of 1,001-1,500 lb Energy Efficient Ice Machine |  |  | $8,835 |
| Average Estimated Purchase Price of 1,001-1,500 lb Energy Efficient Ice Machine** |  |  | $5,301 |
| Average List Price of > 1,500 lb Energy Efficient Ice Machine |  |  | $12,780 |
| Average Estimated Purchase Price of > 1,500 lb Energy Efficient Ice Machine** |  |  | $7,668 |
| *Costs taken from published 2007-2008 manufacturers list prices |  |  |  |
| **Estimated purchase price based on a typical 40% discount from the published list prices |  |  |  |

Equipment Cost Data for CEE Tier III Ice Machines

| Make Energy Efficient | Model | Ice Harvest Rate (lb/24h) | Cost($)* |
| --- | --- | --- | --- |
| Ice-O-Matic | ICE220FA | 175 | $3,510 |
| Hoshizaki | DCM270BAH | 215 | $5,610 |
| Hoshizaki | F-300BAF | 232 | $6,280 |
| Scotsman | CO330MA-1A | 280 | $3,737 |
| Manitowoc | SF-0406A | 331 | $5,059 |
| Hoshizaki | F-500BAF | 359 | $7,310 |
| Scotsman | MDT5N40A | 409 | $9,012 |
| Manitowoc | SY-0504A | 440 | $4,510 |
| Hoshizaki | F-801MAH-C | 552 | $6,550 |
| Scotsman | CO830MA-3A | 724 | $6,749 |
| Manitowoc | SY-0854A | 725 | $6,345 |
| Scotsman | CO1030MA-32A | 844 | $7,560 |
| Scotsman | CO1448MA-3A | 1242 | $8,686 |
| Hoshizaki | KM1601SRH3/URC-21F | 1459 | $9,950 |
| Scotsman | C1848MR-3A/ERC611-32A | 1645 | $10,995 |
| Hoshizaki | F-2000MRH/URC-20F | 1675 | $14,070 |
| Scotsman | FME2404AS-3B | 1800 | $15,613 |
| Average List Price of 101-200 lb Energy Efficient Ice Machine |  |  | $3,510 |
| Average Estimated Purchase Price of 101-200 lb Energy Efficient Ice Machine** |  |  | $2,106 |
| Average List Price of 201-300 lb Energy Efficient Ice Machine |  |  | $5,209 |
| Average Estimated Purchase Price of 201-300 lb Energy Efficient Ice Machine** |  |  | $3,125 |
| Average List Price of 301-400 lb Energy Efficient Ice Machine |  |  | $6,185 |
| Average Estimated Purchase Price of 301-400 lb Energy Efficient Ice Machine** |  |  | $3,711 |
| Average List Price of 401-500 lb Energy Efficient Ice Machine |  |  | $6,761 |
| Average Estimated Purchase Price of 401-500 lb Energy Efficient Ice Machine** |  |  | $4,057 |
| Average List Price of 501-1,000 lb Energy Efficient Ice Machine |  |  | $6,801 |
| Average Estimated Purchase Price of 501-1,000 lb Energy Efficient Ice Machine** |  |  | $4,081 |
| Average List Price of 1,001-1,500 lb Energy Efficient Ice Machine |  |  | $9,318 |
| Average Estimated Purchase Price of 1,001-1,500 lb Energy Efficient Ice Machine** |  |  | $5,591 |
| Average List Price of > 1,500 lb Energy Efficient Ice Machine |  |  | $13,559 |
| Average Estimated Purchase Price of > 1,500 lb Energy Efficient Ice Machine** |  |  | $8,135 |
| *Costs taken from published 2007-2008 manufacturers list prices |  |  |  |
| **Estimated purchase price based on a typical 40% discount from the published list prices |  |  |  |

Equipment Cost Data for Baseline Model Ice Machines

| Make Baseline | Model | Ice Harvest Rate (lb/24h) | Cost($)* |
| --- | --- | --- | --- |
| Scotsman | SCE170A-1# | 110 | $3,293 |
| Hoshizaki | KM-255BAH | 207 | $4,070 |
| Hoshizaki | KML-451MAH | 349 | $3,940 |
| Scotsman | CO522MA-1A | 380 | $4,258 |
| Ice-O-Matic | ICE500FT | 470 | $4,305 |
| Hoshizaki | KM-650MAH | 512 | $4,650 |
| Manitowoc | SD-0872C/CVD-0885 | 600 | $5,653 |
| Manitowoc | SD-1092N3/JC-0895 | 850 | $7,169 |
| Manitowoc | SD-1402A | 1160 | $8,208 |
| Hoshizaki | KM-1300SRH/URC-21F | 1163 | $7,630 |
| Manitowoc | SD-1802A | 1540 | $10,945 |
| Manitowoc | SD-1992N/JC-1395 | 1610 | $10,845 |
| Hoshizaki | KM-1800SRH/URC-20F | 1651 | $10,370 |
| Scotsman | C2148SR-32A/ERC611-32# | 1849 | $13,186 |
| Average List Price of 101-200 lb Baseline Ice Machine |  |  | $3,293 |
| Average Estimated Purchase Price of 101-200 lb Baseline Ice Machine** |  |  | $1,976 |
| Average List Price of 201-300 lb Baseline Ice Machine |  |  | $4,070 |
| Average Estimated Purchase Price of 201-300 lb Baseline Ice Machine** |  |  | $2,442 |
| Average List Price of 301-400 lb Baseline Ice Machine |  |  | $4,099 |
| Average Estimated Purchase Price of 301-400 lb Baseline Ice Machine** |  |  | $2,459 |
| Average List Price of 401-500 lb Baseline Ice Machine |  |  | $4,305 |
| Average Estimated Purchase Price of 401-500 lb Baseline Ice Machine** |  |  | $2,583 |
| Average List Price of 501-1,000 lb Baseline Ice Machine |  |  | $5,824 |
| Average Estimated Purchase Price of 501-1,000 lb Baseline Ice Machine** |  |  | $3,494 |
| Average List Price of 1,001-1,500 lb Baseline Ice Machine |  |  | $7,919 |
| Average Estimated Purchase Price of 1,001-1,500 lb Baseline Ice Machine** |  |  | $4,751 |
| Average List Price of > 1,500 lb Baseline Ice Machine |  |  | $11,337 |
| Average Estimated Purchase Price of > 1,500 lb Baseline Ice Machine** |  |  | $6,802 |
| *Costs taken from published 2007-2008 manufacturers list prices |  |  |  |
| **Estimated purchase price based on a typical 40% discount from the published list prices |  |  |  |

Equipment Incremental Cost Data for Energy Star® Ice Machines*

| 101-200 lb Energy Star® Ice Machine List Price Average Incremental Cost Difference | $315 |
| --- | --- |
| 101-200 lb Energy Star® Ice Machine Estimated Purchase Price Average Incremental Cost Difference** | $189 |
| 201-300 lb Energy Star® Ice Machine List Price Average Incremental Cost Difference | $1,364 |
| 201-300 lb Energy Star® Ice Machine Estimated Purchase Price Average Incremental Cost Difference** | $818 |
| 301-400 lb Energy Star® Ice Machine List Price Average Incremental Cost Difference | $468 |
| 301-400 lb Energy Star® Ice Machine Estimated Purchase Price Average Incremental Cost Difference** | $281 |
| 401-500 lb Energy Star® Ice Machine List Price Average Incremental Cost Difference | $105 |
| 401-500 lb Energy Star® Ice Machine Estimated Purchase Price Average Incremental Cost Difference** | $63 |
| 501-1,000 lb Energy Star® Ice Machine List Price Average Incremental Cost Difference | $389 |
| 501-1,000 lb Energy Star® Ice Machine Estimated Purchase Price Average Incremental Cost Difference** | $233 |
| 1,001-1,500 lb Energy Star® Ice Machine List Price Average Incremental Cost Difference | $916 |
| 1,001-1,500 lb Energy Star® Ice Machine Estimated Purchase Price Average Incremental Cost Difference** | $550 |
| > 1,500 lb Energy Star® Ice Machine List Price Average Incremental Cost Difference | $1,443 |
| > 1,500 lb Energy Star® Ice Machine Estimated Purchase Price Average Incremental Cost Difference** | $866 |
| *Costs taken from published 2007-2008 manufacturers list prices |  |
| **Estimated purchase price based on a typical 40% discount from the published list prices |  |

Equipment Incremental Cost Data for CEE Tier III Ice Machines*

| 101-200 lb CEE Tier III Ice Machine List Price Average Incremental Cost Difference | $217 |
| --- | --- |
| 101-200 lb CEE Tier III Ice Machine Estimated Purchase Price Average Incremental Cost Difference** | $130 |
| 201-300 lb CEE Tier III Ice Machine List Price Average Incremental Cost Difference | $1,139 |
| 201-300 lb CEE Tier III Ice Machine Estimated Purchase Price Average Incremental Cost Difference** | $683 |
| 301-400 lb CEE Tier III Ice Machine List Price Average Incremental Cost Difference | $2,086 |
| 301-400 lb CEE Tier III Ice Machine Estimated Purchase Price Average Incremental Cost Difference** | $1,252 |
| 401-500 lb CEE Tier III Ice Machine List Price Average Incremental Cost Difference | $2,456 |
| 401-500 lb CEE Tier III Ice Machine Estimated Purchase Price Average Incremental Cost Difference** | $1,474 |
| 501-1,000 lb CEE Tier III Ice Machine List Price Average Incremental Cost Difference | $977 |
| 501-1,000 lb CEE Tier III Ice Machine Estimated Purchase Price Average Incremental Cost Difference** | $586 |
| 1,001-1,500 lb CEE Tier III Ice Machine List Price Average Incremental Cost Difference | $1,399 |
| 1,001-1,500 lb CEE Tier III Ice Machine Estimated Purchase Price Average Incremental Cost Difference** | $839 |
| > 1,500 lb CEE Tier III Ice Machine List Price Average Incremental Cost Difference | $2,222 |
| > 1,500 lb CEE Tier III Ice Machine Estimated Purchase Price Average Incremental Cost Difference** | $1,333 |
| *Costs taken from published 2007-2008 manufacturers list prices |  |
| **Estimated purchase price based on a typical 40% discount from the published list prices |  |