Work Paper WPSDGEREHC1060

Revision 3

San Diego Gas & Electric

Energy Efficiency Engineering

Energy Star Room Air Conditioners

# At a Glance Summary

| Measure Name: | Energy Star Room Air Conditioners |
| --- | --- |
| Savings Impacts Common Units: | Per Unit |
| Customer Base Case Description: | 9.15 Mean EER (Current Code Basis) |
| Code Base Case Description: | Same as Customer Base Case |
| Costs Common Units: | Same as Savings Impacts. |
| Measure Equipment Cost ($/unit): | $220 |
| Measure Incremental Cost ($/unit): | $50 |
| Measure Installed Cost ($/unit): | N/A (Assumed self install) |
| Measure Load Shape: | AC_Cooling-RC |
| Effective Useful Life (years): | 9 years |
| Program Type: | Replace On Burnout (ROB) and New |
| TOU AC Adjustment: | 0% |
| Net-to-Gross Ratios: | Res-Default >2 .55 |
| Building Type: | Single and Multi-Family |
| Building Vintage: | All Residential |
| Important Comments: |  |

# At-A-Glance Measure List

| Measure Code | DEER RunID | Measure Description | Building Type | Building Vintage | Climate Zone | Peak Electric Demand Reduction (kW/unit) | Electric Savings (kWh/unit) | Gas Savings (therms/unit) | Base Case Cost ($/unit) | Measure Cost ($/unit) | Measure Incremental Cost ($/unit) | Effective Useful Life (years) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N/A | N/A | Room AC | All Residential | All | 7 | 0.015 | 24 | 0 | $170 | $220 | $50 | 9 |
| N/A | N/A | Room AC | All Residential | All | 10 | 0.063 | 60 | 0 | $170 | $220 | $50 | 9 |

# Document Revision History

# Rev#	         Date		Description                                           Author

| Revision 0 | Jan 8, 2010 | Adopted form SCE WPSCRHC0001. | N/A |
| --- | --- | --- | --- |
| Revision 1 | March 7, 2010 | *Data was updated to reflect SDGE territory climate zone * Data was updated to fit new Residential Retrofit High Impact Measure Evaluation Plan findings | Lucie Sidibe |
| Revision 2 | January 8th 2011 | Cost data was updated to be consistent with Energy Star4, as agreed statewide | Lucie Sidibe |
| Revision3 | June 6, 2012 | Revised NTGR to DEER 2011 | Charles Harmstead (SDGE) |

Note: The information provided in this Work Paper was developed using the best available technical resources at the time this document was prepared.

# Work Paper Approvals

| Harmstead Charles Engineering Supervisor | Date |
| --- | --- |
| Insert Title: | Date |

# Table of Contents

At a Glance Summary	i

At-A-Glance Measure List	ii

Document Revision History	ii

Work Paper Approvals	iii

Table of Contents	iv

Section 1. General Measure & Baseline Data	v

1.1	Measure Description & Background	v

1.4	Measure Effective Useful Live (EUL)	vii

1.5	Net-to-Gross Ratios for Different Program Strategies	vii

1.6	Gross Realization Rate	vii

1.7	Time-of-Use Adjustment Factor	vii

Section 2. Calculation Methods	viii

The CADMUS group Residential Retrofit High Impact Measure Evaluation plan was utilized in order to estimate the savings associated with RAC.	viii

Section 3. Load Shapes	viii

Section 4. Base Case & Measure Costs	viii

4.1 Base Case Costs	ix

4.2 Gross Measure Costs	ix

4.3 Incremental Measure Costs	ix

References	x

# Section 1. General Measure & Baseline Data

## Measure Description & Background

This work paper (WPSDGEREHC1060) details the rationale and saving estimation method for early replacement of residential room air conditioners (RAC) that meet Energy Star requirements. Energy Star Room air conditioners (ES-RAC) are defined as being at least 10% more energy efficient than the minimum federal government standards.

  - DEER Differences Analysis

This specific measure is not included in the Database for Energy Efficient Resources (DEER) Version 2008.2.05.  Therefore, DEER value will not be used in the energy saving and demand reduction estimation. To determine both energy savings and demand reduction, this work paper uses Residential Retrofit High Impact Measure Evaluation Report prepared by the CADMUS group, inc. and it provides RAC estimated annual energy savings and peak demand reduction in climate zone 7 and 10.

  - Codes & Standards Requirements Analysis

Title 20 Appliance Efficiency Regulation: This measure fall under Title 20 of the California energy regulation section 1605.1 (b). As show in table 1 below

Note that, The Minimum EER or COP Effective October 1, 2000 column lists the current code requirements for C-RAC units.

Table 1: 1605.1 (b) Table B-2 Standards for Room Air Conditioners and Room Air-Conditioning Heat Pumps

| Appliance | Louvered Sides | Cooling Capacity (Btu/hr) | Minimum EER or COP |  |
| --- | --- | --- | --- | --- |
|  |  |  | Effective January 1, 1990 | Effective October 1, 2000 |
| Room Air Conditioner | Yes | < 6,000 | 8.0 | 9.7 |
| Room Air Conditioner | Yes | ≥ 6,000 - 7,999 | 8.5 | 9.7 |
| Room Air Conditioner | Yes | ≥ 8,000 - 13,999 | 9.0 | 9.8 |
| Room Air Conditioner | Yes | ≥ 14,000 - 19,999 | 8.8 | 9.7 |
| Room Air Conditioner | Yes | ≥ 20,000 | 8.2 | 8.5 |
| Room Air Conditioner | No | < 6,000 | 8.0 | 9.0 |
| Room Air Conditioner | No | ≥ 6,000 - 7,999 | 8.5 | 9.0 |
| Room Air Conditioner | No | ≥ 8,000 - 19,999 | 8.5 | 8.5 |
| Room Air Conditioner | No | ≥ 20,000 | 8.2 | 8.5 |
| Room Air Conditioning Heat Pump | Yes | < 20,000 | 8.5 | 9.0 |
| Room Air Conditioning Heat Pump | Yes | ≥ 20,000 | 8.5 | 8.5 |
| Room Air Conditioning Heat Pump | No | < 14,000 | 8.0 | 8.5 |
| Room Air Conditioning Heat Pump | No | ≥ 14,000 | 8.0 | 8.0 |
| Casement-Only Room Air Conditioner | Either | Any | * | 8.7 |
| Casement-Slider Room Air Conditioner | Either | Any | * | 9.5 |
| *Casement-only room air conditioners and casement-slider room air conditioners are not separate product classes under standards effective January 1, 1990. Such appliances, if manufactured before October 1, 2000, are subject to the applicable standards in Table B-2 for the other room air conditioners and room air-conditioning heat pumps based on capacity and the presence or absence of louvered sides. |  |  |  |  |

Federal Standard: This measure fall under the Energy Star Room Air Conditioner criteria show in table 2 below

Table2:  Energy Star Qualified Room Air Conditioner (RAC) Eligibility

| Capacity (Btu/Hr) | Federal Standard EER, with louvered sides | ENERGY STAR EER, with louvered sides | Federal Standard EER, without louvered sides | ENERGY STAR EER, without louvered sides |
| --- | --- | --- | --- | --- |
| < 6,000 | >= 9.7 | >= 10.7 | >= 9.0 | >= 9.9 |
| 6,000 to 7,999 |  |  |  |  |
| 8,000 to 13,999 | >= 9.8 | >= 10.8 | >= 8.5 | >= 9.4 |
| 14,000 to 19,999 | >= 9.7 | >= 10.7 |  |  |
| >= 20,000 | >= 8.5 | >= 9.4 |  |  |
| Casement | Federal Standard EER |  | ENERGY STAR EER |  |
| Casement-only | >= 8.7 |  | >= 9.6 |  |
| Casement-slider | >= 9.5 |  | >= 10.5 |  |
| REVERSE CYCLE |  |  |  |  |
| Capacity (Btu/Hr) | Federal Standard EER, with louvered sides | ENERGY STAR EER, with louvered sides | Federal Standard EER, without louvered sides | ENERGY STAR EER, without louvered sides |
| < 14,000 | n/a | n/a | >= 8.5 | >= 9.4 |
| >= 14,000 |  |  | >= 8.0 | >= 8.8 |
| < 20,000 | >= 9.0 | >= 9.9 | n/a | n/a |
| >= 20,000 | >= 8.5 | >= 9.4 |  |  |

## Measure Effective Useful Live (EUL)

Based on DEER 2008, The EUL for this measure is assumed to be 9 years.

## Net-to-Gross Ratios for Different Program Strategies

The NTG values were obtained from the DEER 2011 NTGR Table.

Table 3 Net-to-Gross Ratio for 10-12 Program Cycle

| Measure Name | Delivery Method | Efficiency and Capacity Descriptor | Target Market | NTGR |
| --- | --- | --- | --- | --- |
| REs | Financial Support - Down-Stream Incentive – Deemed | Default | Single Family Or Multi family Markets with Moderate Market Share (>=5%) | 0.55 |

## Gross Realization Rate

The GRR for this measure does not apply.

## Time-of-Use Adjustment Factor

The TOU for this measure is assumed to be 0%

# Section 2. Calculation Methods

## The CADMUS group Residential Retrofit High Impact Measure Evaluation plan was utilized in order to estimate the savings associated with RAC.

According to the reportError: Reference source not found, Energy (kWh) savings and demand (kW) reduction was determined through a combination of on-site metering and lab tests.  Metering was conducted on a total of 102 participant RAC units for 90 days during June through September 2009. Two meters were used for each unit, the Watts up? PRO.Net (recording demand values) and the HOBO U12-12 (recording indoor temperature and humidity). Hourly regression models were developed to estimate hourly energy use as a function of outdoor temperature. Lab tests were conducted for four RACs. This testing was performed to provide the difference in demand between qualifying ENERGY STAR units and standard (Non-ENERGY STAR) units.

Table 4 below, shows the total savings (kWh) annually and the mean peak demand reduction (kW) of going from a standard efficiency RAC to a program qualified ENERGY STAR unit. Also, it summarizes the modeled energy usage (kWh) for Climate Zone 7 and 10.

Table 4: Residential Retrofit HIM Evaluation report Room AC Estimated Annual Energy and Peak Demand Savings for Climate Zone 7 and 10

| Climate Zone | Modeled Energy Usage (kWh) | Total Unit Energy Savings During Cooling Season (kWh) | Mean Unit Energy Peak Demand Savings (kW) |
| --- | --- | --- | --- |
| 7 | 240 | 24 | 0.015 |
| 10 | 592 | 60 | 0.063 |

# Section 3. Load Shapes

The table below shows the assumed load shape

Table 5: RAC Load Shape

| Target Sector (Actual) | Target Sector (Alternate) | Load Shape |
| --- | --- | --- |
| Residential | Residential – Single Family | AC_Cooling-RC |

# Section 4. Base Case & Measure Costs

- Because DEER 2008 does not contain completed IMC data for room A/Cs of a lower EER (below 11), this work paper relies on the Energy Star data to determine the base case and measure cost. This work paper also considers the installation cost differential between the base case and measure case to be zero for the customer.

- .

## 4.1 Base Case Costs

Energy Star considers the existing equipment to be a Conventional Unit, Manufactured After 1994. The cost is estimated to be $170 per unit.

## 4.2 Gross Measure Costs

Similarly, Energy consider the measure cost to be $220, according to energy star, this data was obtained from the average retail price of a qualified model in 2008 from national retail data.

## 4.3 Incremental Measure Costs

The only cost differences are the extra capital costs of purchasing an Energy Star unit over a non-energy star unit.  Thus the incremental cost is estimated at $50

# References

Database for Energy Efficient Resources (DEER) Version 2008.2.05 at www.deeresources.com

Residential Retrofit High Impact Measure Evaluation Report, the CADMUS group Inc., December 7, 2009

3 DEER2011 NTGR